Cape Lambert A Switchgear Replacement Project Health, Safety and Environment

Management Plan

Document ID: 201106-SE-HSE-0001

Authority

|  | Title | Name | Date |
| --- | --- | --- | --- |
| Owner | Project Manager | Jordan Ennis | 14/04/2026 |
| Reviewer | HSEQ Manager | Anthony Gollan | 14/04/2026 |
| Approver | Project Manager | Jordan Ennis | 14/04/2026 |

History

| Revision | Date | Amended By (Name) | Details of Amendment |
| --- | --- | --- | --- |
| A | 14/04/2026 | Anthony Gollan | New Project Plan |

Table of Contents

1	General	8

1.1	Scope of Work	8

2	HSE Policies	9

3	Definitions	10

4	Planning	12

4.1	Operational Risk Management	12

4.1.1	Level 2 Risk Assessment Workshop (L2RA)	13

4.2	Legal and Other Requirements	14

4.3	Health, Safety, Environment Management Plan	15

4.3.1	Plan Intent	15

4.3.2	Plan Issue and Authority	15

4.3.3	Plan Objectives	15

4.3.4	Targets and Key Performance Indicators (KPI’s)	16

5	Implementation	17

5.1	Culture	17

5.2	Security	18

5.3	Safety Requirements for Purchasing	18

5.4	Registers	19

5.5	Preventative Maintenance and Inspections	19

5.6	Subcontractors	19

5.7	HSE Management System Structure	20

5.8	Document and Data Control	21

5.9	Records and Records Management	21

5.10	Responsibility and Accountability	22

5.10.1	HSE Responsibilities for All SCEE Workers	22

5.10.2	Project Manager	23

5.10.3	HSE Advisor	24

5.10.4	Construction Manager/Supervisors	25

5.10.5	Health and Safety Representatives	26

6	Personnel Management	26

6.1	Worker and Sub-Contractor Selection and Mobilisation	26

6.2	Pre-Employment Medicals	27

6.3	Induction, Training and Competency	27

6.4	4WD Training	28

6.5	SCEE Site Specific Induction	28

6.6	Area Specific Inductions	29

6.7	E-Learning Portal	29

6.8	SCEE First Swing Orientation Program	29

6.9	Visitor Induction and Controls	29

6.10	Fitness for Work (FFW)	30

6.11	HSE Discipline	30

6.12	Verification of Competency and Re-Training Frequency	31

7	Consultation and Communication	31

7.1	Pre-Start Meetings	31

7.2	Refocus Meeting	32

7.3	Toolbox Meetings	32

7.4	HSE Promotion	33

7.5	Project Safety Committee	33

7.6	Management Meetings	34

7.7	HSE Notice Boards	34

7.8	Safety Incentives	34

7.9	HSE Alerts and Information	35

7.10	Health and Safety Issue Resolution	35

7.11	Signage	36

8	Hazard and Risk Management	36

8.1	General	36

8.2	Hazard Identification	36

8.3	Hazard Observation Reports (HazOb)	37

8.4	Hazard and Risk Assessment	37

8.5	Risk Control	37

8.6	Work Instructions	38

8.7	Safe Work Method Statements (SWMS)	38

8.8	Take 5	39

8.9	Job Hazard Analysis (JHA)	39

9	Control of Hazards / Risks	41

9.1	5 Star Commitment	41

9.2	Critical Risk Management (CRM)	41

9.3	Family Star	43

9.4	Fatality Prevention Program (FPP)	43

9.5	Stop Work Authority	44

9.6	Mobile Phones	44

9.7	Work Environment and Housekeeping	45

9.8	Personal Protective Equipment (PPE)	46

9.8.1	Personal Adornments	46

9.9	Working in Heat	47

9.10	Hydration Testing	47

9.11	Permits to Work	48

9.12	Working at Heights	48

9.12.1	Falling Object Prevention and Encapsulation	49

9.13	Working from Platform Ladders	49

9.14	Working Over/Near Water	50

9.15	Excavations and Penetrations	51

9.16	Isolation of Plant and Equipment	51

9.17	Hot Work	52

9.18	Drilling into Cable Ladder and Panels	52

9.19	Machinery and Vehicle Inspection	53

9.20	Driving	53

9.21	Cranes, Lifting and Rigging Equipment	54

9.22	Forklift/Telehandler Operations	56

9.23	Loading and Unloading	56

9.24	Scaffolding and Static Lines	57

9.25	Barricading	57

9.26	Inclement Weather	59

9.27	Traffic Management	59

9.28	Use of Power and Hand Tools	59

9.28.1	Electric Arc Welding	60

9.28.2	Hand Held Drills	60

9.28.3	Hand Held Grinders	61

9.28.4	Rota Broach/Magnetic Drills	61

9.29	Banned Tools Onsite	62

9.30	Plant and Equipment – Other	62

9.30.1	Installation/Commissioning/Demolition	62

9.30.2	Fit for Purpose	63

9.30.3	Plant and Equipment Controls	63

9.30.4	Maintenance/Inspection of Plant	63

9.31	Fire Prevention	63

9.32	Electrical Safety	64

9.32.1	Electrical Isolations	64

9.32.2	High Voltage Work	65

9.32.3	Electric Shocks and Events	66

9.32.4	Electrical Fires	67

9.32.5	Controlling Electrical Hazards	67

9.33	Substations	68

9.34	Noise Control and Occupational Hygiene Monitoring	68

9.35	Health Surveillance	68

9.36	Fibrous Materials	69

9.36.1	Hazardous Paint Removal	69

9.37	Working Alone	70

9.38	Simultaneous Work Activities	71

9.39	Management of Change (MOC)	71

9.40	Hazardous Substances	72

9.41	Risk Assessment and Control of Hazardous Substances	72

9.42	Storage and Disposal of Hazardous Materials	73

9.43	Hydrocarbon and Other Chemical Management	73

9.44	Hydrocarbon and Chemical Spill Management	74

9.45	Pollution /Emissions	75

9.46	Dust Management	75

9.47	Protection of Flora and Fauna	75

9.48	Soil and Water Conservation	76

9.49	Waste Water Management	76

9.50	Waste Reduction and Disposal	76

9.51	Weed Management	77

9.52	Surface Water/ Ground Water Management	78

9.53	Indigenous Culture and Local Community Partnership	78

9.54	Rehabilitation	78

10	Emergency Preparedness and Response	79

10.1	Cyclone Preparedness and Response	79

11	Incident Management and Response	81

11.1	First Aid and Medical Treatment	81

11.2	Critical Incident Management	81

11.3	Injury Management and Rehabilitation	82

11.4	Incident Reporting	82

11.5	Incident Investigation	83

12	Monitoring, Auditing and Review	84

12.1	Daily Safety Report	84

12.2	Monthly HSE Performance Report	84

12.3	Workplace Inspections	85

12.4	Audits	85

12.5	HSE Non-Conformance and Corrective Actions	86

12.6	Project Close Out Report	87

13	Management Review	87

13.1	HSE Management System Review	87

13.2	Executive Management Review	88

Appendix A - Responsibility Statements	89

# General

## Scope of Work

This HSE Management Plan has been developed for the Cape Lambert A Switchgear Replacement Project.  The Project is located at CLA Port (Site). The Site is approximately 40 km from the town of Karratha in the Pilbara region of Western Australia.

Site Location Map

Four (4) existing air insulated 6.6kV switchboards, located at the CLA sites listed below, are experiencing premature failures and have been assessed as non-compliant with the current RTIO standards:

- Wharf Sub 1 (WFSUB1) 6.6kV LC71, Nilsen make, manufactured 2008.

- Wharf Sub 2 (WFSUB2) 6.6kV LC79, Nilsen make, manufactured 2008.

- Ship loader 3 (SL03) 6.6kV LC73, Nilsen make, manufactured 2008; and

- Reclaimer 4 (RC04) 6.6kV LC72, Nilsen make, manufactured 2008.

As part of this project, the existing switchboards identified above will be replaced with new 6.6kV Siemens NXPlus C GIS switchboards. Additionally, existing remote switching panels will be upgraded with new units to align with the updated system architecture.

Based on the current asset locations and design, no deconstruction or installation activities are anticipated to occur over water.

SCEE Scope of Work includes the provision of electrical and instrumentation works including supply of labour, materials, supervision, plant, services, equipment and all other things that are or may be required to be performed by SCEE under and in accordance with the Scope of Work document P-0603621-4850-E-SOW-00002.

The purpose of this document is to clearly define a central and effective standard of strategies, systems and responsibilities to be applied throughout the Project for effectively managing safety and health.

# HSE Policies

SCEE aspires to achieving a workplace free from damage or harm to people, equipment and the environment and policies to govern safety, business conduct, social, environmental and economic activities are integral to our activities.

SCEE adopts six key policies to drive the sustainable development and application of HSE Management across our operations consisting of:

Health and Safety Policy

Environmental Policy

Fitness For Work Policy

Working Environment Awareness Policy

Workplace Rehabilitation Policy

Non-Smoking Policy

These policies are reviewed annually or whenever there is a significant change to SCEE operations or legislative requirements.

The policies are approved and signed by the CEO/Managing Director and are displayed in all SCEE offices and crib huts and communicated to all workers and supervisors at SCEE inductions.

SCEE Project Management shall ensure the requirements of all HSE policies are communicated to all personnel.

Refer:	SCEE-BS-HS-POL-0001 Health and Safety Policy

SCEE-BS-HS-POL-0002 Environmental Policy

SCEE-BS-HS-POL-0004 Working Environment Awareness Policy

SCEE-BS-HS-POL-0005 Non-Smoking Policy

SCEE-HR-HR-POL-0004 Fitness for Work Policy

SCEE-HR-IM-POL-0001 Workplace Rehabilitation

# Definitions

| Term / Acronym | Definition |
| --- | --- |
| 3WLA | Three (3) Week Look Ahead |
| ACA | Australasian Corrosion Association |
| AFC | Approved for Construction |
| ALARP | As Low as Reasonably Practicable. A risk management principle based on reducing risk. |
| AR | Approval Request |
| AS / NZS | Australian / New Zealand Standards |
| Aspect | The element of activities, product or service that can affect the environment |
| BAC | Blood Alcohol Content |
| CAR | Corrective Action Report |
| CDS | Contractor Data Schedule |
| CLA | Camp Lambert Port A |
| CLB | Camp Lambert Port B |
| Competent Person | Refers to a person who has acquired through training, qualification or experience, the knowledge and skills required to do a designated task competently. |
| Consequence | The outcome of an event or situation expressed as the extent of harm. |
| Controls | Measures that reduce or eliminate a hazard. |
| COS | Coarse Ore Stockpile |
| CPR | Cardio Pulmonary Resuscitation which is an emergency medical procedure for a victim of cardiac arrest or, in some circumstances, respiratory arrest. |
| CRA | Construction Risk Assessment |
| Critical Incident | Any situation faced by an individual that causes him or her to experience unusually strong emotional and/or physical reactions that can interfere with their ability to productively carry on with their everyday lives and which has a detrimental impact on the efficient productive operations of the Project. Also considered to be any situation that could adversely affect SCEE or the Client, such as public perception and/or outrage. |
| CRM | Critical Risk Management |
| CV | Conveyor |
| DEMIRS | Department of Energy, Mines, Industry Regulation and Safety |
| DWER | Department of Water and Environmental Regulation |
| EAP | Worker Assistance Program |
| EIC | Electrical, Instrumentation and Controls |
| EMS | Environmental Management System |
| ERT | Emergency Response Team |
| Evaluate | To ascertain or set up the amount or value |
| Event | A collective term which includes instances of actual or potential injury to SCEE workers and/or contractors, damage to plant and equipment or the environment |
| FAC | First Aid Case. |
| FAT | Factory Acceptance Testing |
| FFW | Fitness For Work |
| Fibrous Material | Any asbestos cement product or asbestos in the natural geology. |
| GET | Ground Engaging Tools |
| GPO | General Purpose Outlet |
| GPS | Global Positioning System |
| Hazard | A source or a situation with a potential for harm in terms of human injury or ill-health, damage to property, damage to the environment or a combination of these. (AS4801) |
| Hazardous Substance | A substance that following worker exposure can have adverse effect of health. A substance meeting the classification criteria specified in the Approved Criteria for Classifying Hazardous Substances (NOHSC: 1008 (2004)). |
| HSE | Health, Safety and Environment |
| HSEMS | Health, Safety and Environmental Management System |
| HSMS | Health and Safety Management System |
| HV | High Voltage |
| Impact | Any change either adverse or beneficial resulting from Impact resulting from an aspect |
| Incident | Any occurrence that may have had the potential to cause illness or damage to persons, property, plant or the environment. |
| IVMS | In Vehicle Monitoring System |
| JHA | Job Hazard Analysis |
| Job Hazard Analysis (JHA) | An analysis of a job or task to identify job steps, hazards and control measures to reduce risks to acceptable levels. |
| LiF | Leadership in the Field |
| LoM | Life of Mine |
| LTI | Lost Time Injury. |
| LTIFR | Lost Time Injury Frequency Rate |
| LV | Low Voltage |
| Manual Handling | Any activity requiring the use of force exerted by a person to lift, lower, push, pull, carry or otherwise move, hold or restrain a person, animal or thing. |
| MEWP | Mobile Elevated Work Platform |
| MoC | Management of Change |
| MTI | Medical Treatment Injury |
| MTIFR | Medical Treatment Injury Frequency Rate |
| NCR | Non-Conformance Report |
| OLC | Over Land Conveyor |
| PEFA | Pre-employment Functional Assessments |
| PMO | Pilbara Mine Operations |
| PPE | Personal Protective Equipment |
| PPU | Protected Premises Unit |
| RCD | Residual Current Device |
| Risk | In relation to any potential injury or harm, the likelihood and consequence of that injury or harm occurring. (AS4801) |
| RTIO | Rio Tinto Iron Ore |
| RWC | Restricted Work Case |
| SCEE | SCEE Electrical Pty Ltd |
| SDS | Safety Data Sheet (formerly MSDS) |
| Shall | Shall is to be understood as mandatory. |
| Should | Should is to be understood as recommended but not mandatory |
| SMP | Safety Management Plan |
| Sus Cap | Sustaining Capital |
| SWMS | Safe Work Method Statement |
| TCCC | Transfer of Care, Custody and Control |
| WI | Work Instruction |
| WLL | Working Load Limit (Rated Capacity (formerly SWL)) |
| Worker | A person is a worker if the person carries out work in any capacity including work as — - an employee; or - a contractor or subcontractor; or - an employee of a contractor or subcontractor; or - an employee of a labour hire company who has been assigned to work in the person’s business or undertaking; or - an outworker; or - an apprentice or trainee; or - a student gaining work experience; or - a volunteer; or a person of a prescribed class. |

# Planning

## Operational Risk Management

It is a requirement that all SCEE operational sites implement HSE hazard and risk management processes across their scope of work, and that those processes are applied to all activities that SCEE can control or influence.

HSE hazards, risks and opportunities must be assessed, prioritised, and managed as appropriate to the nature, scale and potential impact of operations and activities both individually and as a whole.

The areas of concern that the risk management process should control are:

Protecting the health and safety of the public

Protecting the health and safety of all personnel

Protecting the environment

Protecting plant, equipment, and property

Protecting the business

SCEE‘s systematic risk management process provides assurance that all planned objectives will be achieved within an acceptable level of risk. The SCEE Risk Management framework is designed in 5 steps supported by consultation /communication and monitoring / review of each step-in line with continual improvement methods.

Refer: 		SCEE-BS-RM-PRO-0001 Corporate Risk Management Procedure

SCEE-BS-RM-PRO-0002 Operational Risk Procedure

Figure 1: Risk Management Framework

### Level 2 Risk Assessment Workshop (L2RA)

Prior to project mobilisation a Level 2 Risk Assessment Workshop (L2RA) will be convened. The risk assessment workshop will identify all aspects of the work scope, identify hazards and risks and detail specific controls to eliminate or minimise the risk using the ALARP principle.

In most states, statutory requirements exist for the control of known hazards. Examples of these are:

- Approved Codes of Practice

- advisory standards

- safety data sheets

- guidance notes

- Australian Standards

Where the above documents exist, the controls referenced in those documents shall be incorporated into the risk assessment.

The L2RA will be documented using the RTIO’s Risk Assessment Template or equivalent SCEE specified document.

The risk assessment shall be reviewed following mobilisation to reflect a greater understanding of the environment and work scope. The CRA shall become the projects hazard and risk register and be maintained for the life of the project. The CRA and Environmental Aspect and Impact register shall be displayed in all offices and crib huts.

Refer:	SCEE-BS-RM-TEM-0006 Construction Risk Assessment

SCEE-BS-RM-PRO-0002 Operational Risk

SCEE-BS-RM-TEM-0002 Risk Criteria Matrix

SCEE-BS-HS-TEM-0016 Environmental Aspect & Impact Register

RTIO-HSE-0062207 Hazard Identification and Risk Management Work Practice

RTIO-HSE-0062204 Level 2 Risk Assessment Work Practice

## Legal and Other Requirements

SCEE Management shall be aware of the legislative obligations applicable to this Project and the importance of complying and exceeding minimum requirements where practicable. These obligations include the conditions of regulatory approvals as well as applicable legislation. All personnel will be made aware of their duty of care obligations and other key areas of legislation applicable to the Project during the site induction.

Legislation relevant to this project may include but is not limited to the following:

| Jurisdiction | Legislative Reference |
| --- | --- |
| WA | - Work Health and Safety Act 2020 |
| WA | - Work Health and Safety (Mines) Regulations 2022 |
| WA | - Explosive and Dangerous Goods Act 1961 |
| WA | - Dangerous Goods Safety Act 2004 |
| WA | - Electricity (Licensing) Regulations 1991 |
| WA | - Environmental Protection Act 1986 |
| WA | - Environmental Protection Regulations 1987 |
| WA | - Code of Practice – Prevention of Falls at Workplaces |
| WA | - Code of Practice – Safeguarding of Machinery and Plant |
| WA | - Code of Practice – Working Hours Risk Management Guidelines |
| WA | - Code of Practice – Violence, Aggression and Bullying at Work |
| WA | - Code of Practice – Excavation |

SCEE Management shall be responsible for making all staff and subcontractors aware of all the requirements of the SCEE HSE Management Plan, procedures, specifications and all legal and other requirements. SCEE Project Management shall be responsible for ensuring that all, personnel including subcontractors adhere to these requirements when performing work.

SCEE Management and Supervision personnel associated with the project shall ensure they have a working knowledge of legal and other requirements applicable to their work scope and work location. SCEE Management shall maintain and provide timely access to applicable legislation and other reference documents when the information is required or requested.

Refer:	SCEE-BS-HS-PRO-0019 HSE Compliance Obligations Procedure

## Health, Safety, Environment Management Plan

### Plan Intent

This HSEMP has been developed to ensure SCEE’s activities on the project are conducted in a manner that protects the health safety and environment of all personnel, including contractors and visitors and maintain compliance with all legal and regulatory requirements. This HSEMP details the actions that are required to manage our HSE obligations and requirements and incorporates the specific requirements of:

- Relevant Legislation, Code of Practice, Standards, and Industry requirements

- SCEE HSE Management System requirements and performance expectations

- Client HSE Management System requirements and performance expectations

The success of the Project depends upon the combined capability and contribution of all persons employed on the Project. SCEE is dedicated to fostering a work environment that challenges, enriches, and rewards each individual.

### Plan Issue and Authority

This HSEMP is a mandatory document under the SCEE HSEMS and a deliverable for the Cape Lambert A Switchgear Replacement Project. The HSEMP is issued under the authorisation of the Project Manager and SCEE Corporate HSE Department and the procedures, practices and methods of control defined herein must be adhered to by all personnel and cannot be formally amended without the agreement of the Corporate HSE Department and Project Manager.

This project HSEMP will be reviewed within 2 months of project mobilisation and site works commencing.

The project HSEMP will be subject to periodic review and when required, amended to reflect changes in contractual or management requirements or to correct any disparity identified during monitoring, auditing or review activities.

### Plan Objectives

The objectives of this HSEMP are to:

Provide a uniform approach to the management of HSE requirements across all SCEE work locations.

Define responsibilities and accountabilities for all personnel to ensure effective implementation of the HSE Management System

Establish and support relationships between the HSE management system, site procedures and relevant standards.

Pro-actively facilitate communication with consultation and participation by all managers, supervisors, and personnel in HSE matters relevant to their role.

Identify, assess, and control all workplace hazards and risks within our control.

Ensure processes for the effective management and reporting of HSE incidents and effective management and rehabilitation of injured personnel.

Facilitate the continual improvement of HSE performance and standards of managers, supervisors, and all personnel.

Provide information for personnel at all levels to meet responsibilities and deliverables.

Facilitate methods to achieve and improve, goals, targets, KPI’s and objectives.

### Targets and Key Performance Indicators (KPI’s)

The SCEE Project HSE targets are:

| Performance Objective | Source Document | KPI | Target |
| --- | --- | --- | --- |
| Maintain compliance with all SCEE and Client HSE management plans and procedural requirements throughout the Contract. | SCEE Project HSEMP Client Project HSEMP Daily Safety Report | Results from SCEE and Client audits. SCEE and Client HSE Reports | Min. of 85% compliance on any audit 100% compliance with HSE reporting requirements HSE/PM/Superintendent – 1 Weekly Site Inspection Supervisors - 1 Star Inspection per day HSE – 1 Star Inspection per day |
| Maintain compliance with inspection and audit schedules. | SCEE-BS-HS-TEM-0015 SCEE-BS-HS-STR-0003 HSE Audit Schedule Corrective Action Register / STEMs | Percentage of HSE inspections completed. Percentage of audits completed. Percentage of Non-conformances & observations closed out within the agreed timeframe. | 100% of inspections conducted to schedule 100% of audits conducted to schedule 100% Non-conformances & observations closed out within agreed time frames |
| Ensure all personnel are adequately trained and competent for the work they are to undertake. | Project Training Plan/Matrix Project Training Matrix / STEMs Project Training Matrix / STEMs | Percentage of personnel trained according to the project training plan Percentage of personnel performing work, for which they are not trained, licensed or verified as competent Percentage of personnel trained in Senior First Aid | 90% of training conducted to schedule 0% of personnel not trained licensed or VOC’d to perform role tasks 5% of all personnel trained in Senior First Aid |
| Achieve the Contract goal of a workplace free from harm and damage to the health & safety of personnel, the environment and equipment. | SCEE-BS-HS-STR-0001 Objectives and Targets | No. of Hazards reported by SCEE & it’s subcontractors No. of injuries reported for SCEE & its subcontractors No. of incidents reported for SCEE & it’s subcontractors No. of environmental incidents reported. No. of P&E damage events reported | 3 HazOb reports per site worker per week, 1 per site Management per week, 5 per site HSE position per week Nil MTI, RWC or LTI injuries Investigations completed within 7 days (exception for TapRooT / ICAM Investigations) Nil reportable environmental incidents |
| Maintain strong environmental performance standards. | SCEE-BS-HS-STR-0001 Objectives and Targets SCEE Project HSEMP Client Project HSEMP | No. Environmental Toolbox Topics No. of SWMS / JHA’s with environmental issues considered No. environmental incidents reported | 2 x Environmental Toolboxes 100% of SWMS/JHA’s with environmental issues/controls included. Nil significant environmental incidents |

SCEE Executive Management develops and endorses HSE Objectives and Targets annually across the business to drive and improve HSE performance.  These are available on the SCEE Intranet and progress against these targets is monitored and communicated periodically to site.

Refer:	SCEE-BS-HS-STR-0001 HSE Objectives and Targets

# Implementation

Project Management shall ensure all workers are informed of their individual responsibilities and duties under this Plan. This shall form part of the formal induction program implemented on site.

Key personnel shall sign responsibility statements to acknowledge an understanding and commitment to their responsibilities under this HSEMP. Responsibility statements to be signed by key project personnel are incorporated in this HSEMP as Appendix A.

HSE Packs shall be established in the field containing relevant information for the project.  The packs as a minimum shall include the HSE Management Plan, CRAW, Relevant Procedures and Work Instructions.  These packs shall be located at each work front and made available to all SCEE workers and client when requested.

Refer:	Appendix A - Responsibility Statements

## Culture

Project Management will create and sustain a culture that supports the objectives of the Health and Safety Management System (HSMS), based on:

- A belief in the company’s desire to improve HSE performance

- Motivation to improve personal HSE performance

- Acceptance of individual responsibility and accountability for HSE performance

- Participation and involvement at all levels in HSEMS development

- A commitment to an effective HSEMS

The implementation of safety and health management on the project shall be in accordance with a fair and just approach. At risk behaviours are not to be tolerated and the proactive re-enforcement of positive behaviours shall be a focus on the project.

## Security

SCEE Project Management shall implement a security program that consists of the following:

Only nominated workers (nominated by the Project Manager or designated persons) have access to office, storage areas and material and equipment sites

All offices, storage areas, material and equipment sites shall be kept locked after hours with keys issued to nominated persons

All personnel shall be made aware at their induction of the requirement to secure and return all plant equipment and materials to designated storage locations at the end of each workday

Supervisors shall ensure that there are no photographs taken of any part of the project that may conflict with legal obligations or requirements regarding the media or local community.

## Safety Requirements for Purchasing

Equipment and material specifications should be in place for all types of plant, equipment, and materials to ensure compliance with the relevant site and legislative requirements and good industry practice.

Equipment and material specifications shall be clearly communicated to procurement personnel to ensure that equipment and material purchases (including chemicals) satisfy   SCEE, Regulatory and Client specifications.  Information concerning prohibited tools shall be provided to the procurement department.

All chemicals that are purchased by SCEE must be approved by the Client before being used on site, have an up-to-date Safety Data Sheet (SDS) and be stored appropriately on site.

Formal inspections of all equipment and materials with the potential to impact worker safety or health shall be conducted when it arrives at a project to check compliance against the purchasing specifications.  Where appropriate, consideration shall also be given to using alternative equipment or materials, which are safer and less hazardous than those being currently used.

Refer:	RTIO-HSE-0101789 Contractor Management – Supply of Equipment and Materials

## Registers

Compliance registers shall be maintained for all relevant activities in accordance with applicable health and safety legislation and site requirements.

The below registers will be maintained in Stems

| Register Name | Comments |
| --- | --- |
| Induction, Training and Competency | Project Training Matrix |
| Actions Register | Maintained in STEMS |
| Hazard Register | Maintained in STEMS |
| Event Register | Maintained in STEMS |

Refer:	SCE-BS-HS-TEM-0065 SCEE Electrical Master HSE Registers

## Preventative Maintenance and Inspections

A complete list of all mobile and portable plant on the Project will be maintained in the SCEE asset management system. This list will form the basis of the formal maintenance log and maintenance, and inspections will be scheduled as a preventative measure for plant and vehicles.

The maintenance system will be supported daily through the mandatory pre-start checks required to be completed by all operators. The overall preventative maintenance process is actioned through:

Vehicle maintenance & Inspections

Plant maintenance & Inspections

Quarterly Inspection and tagging of electrical, lifting equipment, ladders, harnesses and lanyards

RCD Operational Checking

Pre-start Checks on all vehicles, plant and equipment

Fire Extinguisher audits

Training needs analysis

Copies of all inspections are recorded in the asset management system, which the Site Manager and HSE Advisor can review at any time and, in collaboration with the site Supervisors, assess the upcoming impact of maintenance activities on the work activities and schedule according to priorities and legislative compliance.

## Subcontractors

In assessing and awarding subcontracts the SCEE Project Manager / Project Supervisor  shall consider all aspects of proposed work methods, materials and manpower to ensure that the subcontractor is capable of performing the work in a safe manner and without risk to its own workers SCEE personnel or others on the project.

Subcontractors shall:

Participate in the SCEE and client inductions prior to commencing work on the project

Provide copies of current insurances i.e., workers compensation, public liability, motor vehicle registration and licences

Liaise with the SCEE Project Manager / Supervisor on all aspects of their work to ensure smooth integration and prevention of conflict with SCEE work personnel and any subcontractors

Participate in Prestart and Toolbox meetings and any general HSE training conducted while on the project as applicable

Provide safety data sheets (SDS) for all chemicals and hazardous materials it plans to use, discuss methods of storage, use training and safety precautions and obtain approval from the SCEE Project Manager / Project Supervisor before bringing the material onto the project

Provide information on quantities of all hazardous materials on the project site for inclusion in the SCEE hazardous substance register

Report all incidents to the SCEE Project Manager / Project Supervisor

Cooperate with SCEE project team in all matters relating to safety and health and comply with all reasonable instructions from the SCEE Project Manager / Project Supervisor or their delegated representative

Sign and abide by the SCEE sub-contractor agreement

- SCEE subcontractors shall be required to complete SCEE site specific inductions, and any client specified inductions prior to commencing work on the project.

- SCEE will supervise all subcontractors being brought to site and will ensure they are non-working supervisor/s in accordance with the Department of Mines, Industry Regulation and Safety supervision guidelines.

Refer:	SCEE-BS-PRO-PRO-0004 Supplier and Subcontractor Selection and Review

SCEE-OP-OP-PRO-0010 Subcontractor Management

SCEE-BS-PO-TEM-0017 Subcontractor Questionnaire

SCEE-OP-OP-LIS-0001 Subcontractor Management Checklist Structure and Responsibility

RTIO-HSE-0101789 Contractor Management – Supply of Equipment and Materials

## HSE Management System Structure

SCEE’s HSE management system complies with the requirements of AS/NZ ISO 9001, AS/NZ ISO 14001 and AS/NZ ISO 45001. The HSE Management System is hierarchical, where documentation is developed to have a relationship and support the requirements of the levels above.

## Document and Data Control

All SCEE HSE documentation and support material can be accessed on the SCEE Intranet.

All project documentation shall be established, recorded, controlled and maintained in accordance with SCEE Document and Data Control procedures.

The Project HSE Advisor is responsible for the management of HSE records and associated documents, and these documents shall be accessible for audit purposes.

Refer:	SCEE-BS-PC-PRO-0010 Project Document and Data Control

## Records and Records Management

The Project Manager and Project HSE Advisor are responsible for the management of records and documents associated with HSE requirements and activities of the project.

Safety records shall be stored and maintained in such a way that they are readily retrievable for audit purposes and protected against damage, deterioration, or loss. Records to be retained include, but should not be limited to the following:

Client Communications

Competency Records

HSE Meeting Minutes

Training Records

Incident Reports and Investigations

Workplace inspections

Plant & Equipment inspections

HSE Reports

Permits / SWMS/ JHA’s

Risk Assessments

Safety and Health Management Plans

Corrective action register

Audits

All HSE records are to be retained and archived in accordance with the SCEE quality management system requirements. As a minimum all records are to be maintained for the life of the project plus retention periods determined by record management procedures and / or in accordance with requirements determined by legislation.

HSE records shall be provided to the RTIO Project Advisor for retention in accordance with RTIO document retention requirements.

Refer: 	SCEE-BS-PC-PRO-0007 SCEE Project Data Numbering Procedure

SCEE-BS-PC-LIS-0004 SCEE Project Documents Overview

## Responsibility and Accountability

### HSE Responsibilities for All SCEE Workers

All SCEE workers (including subcontractors) shall assist in the prevention of events by:

Being responsible for their own safety and health and that of others in the workplace

Showing an understanding and demonstrating a commitment to preventing injuries, minimising damage and avoiding interruption to the organisation

Keeping the workplace in a clean and tidy condition and not interfering with or misusing protective equipment (personal or mechanical) that has been provided or installed for the purpose of safety and health

Using all personal and mechanical protective equipment issued or supplied as per the workplace requirements

Attending prestart meetings and participating in pre-start tasks

Attending safety presentations and toolbox meetings

Complying with the requirements for the protection of the environment

- Being responsible for their own environmental impact and that of others in the workplace

- Comply with the requirements of the RTIO Critical Risk Management process

- Demonstrate environmental awareness

Working in accordance with SCEE and requirements that have been produced to ensure the protection of the environment

Immediately reporting all events, incidents or unsafe conditions that occur in the workplace

Working in accordance with SCEE procedures and requirements that have been produced to ensure the safety and health of all workers and others

Complying with the requirements of all statutory HSE legislation where applicable

Participating in Fitness for Work testing as requested

Adhering to permit requirements

Assisting management in achieving the project HSE goals and objectives

Immediately stopping any “Unsafe Behaviour” identified during daily work activities

Understanding the Heath, Safety and Environment Management Plan and Quality Plan as it relates to their position

Refer:	RTIO-HSE-0075146 Organisational Resources, Accountabilities and Responsibilities

### Project Manager

Reports to the Chief Operating Officer, in SCEE Head Office and is accountable for:

The overall safety performance of all SCEE workers and subcontractors under their control

Ensuring that the necessary procedures are in place to carry out all aspects of the Project Safety Management Plan

Ensuring that all requirements of this HSEMP, and HSE procedures are fully implemented and complied with

Providing adequate resources for the implementation of the HSEMP

Reporting all project progress in relation to safety to SCEE Head Office

Motivating SCEE staff to pursue their individual responsibilities within the management plans

Leading by example and conducting regular site inspections to assess SCEE’s compliance to the HSEMP requirements

Supporting SCEE Management and the Client in the resolution of all health and safety issues

Reviewing event and incident reports and minutes of safety meetings

Reviewing environmental protection activities including waste disposal

Reviewing HSE performance at regular intervals and following up any identified issues

Reviewing the implementation of the HSEMP

Comply with the requirements of the RTIO Critical Risk Management process

Being visible on site to encourage, monitor and assess all aspects of safe working procedures

Managing all sub-contractors engaged in their scope of works as SCEE workers

Ensuring the provision of adequate health and safety resources, services and facilities to effectively implement and manage the health and safety program

Reviewing Project HSE performance weekly (lag and lead indicators) with the HSE Team

Participating in Project HSE systems audits as scheduled

Ensure weekly toolbox meetings are undertaken

Undertaking Project field inspections

Encouraging proactive near-miss and hazard reporting by all site personnel

Addressing any hazards and risks identified during daily work activities

Immediately stopping any “Unsafe Behaviour” identified during daily work activities

Refer:	RTIO-HSE-0075146 Organisational Resources, Accountabilities and Responsibilities

### HSE Advisor

The HSE Advisor reports to the Project Manager/Project Supervisor and has functional reporting requirements to the HSEQ Manager.  Responsibilities and accountabilities include:

Advising Project Management in HSE matters and ensuring the project complies with all health and safety requirements

Assisting with the review and approval of the HSEMP

Developing and delivering training programs to project personnel in risk management for potential hazards on the project site

Initiating and developing safety training to assist with cultural change

Making recommendations and contributing to improving SCEE’s HSE system

Preparing HSE performance reports for SCEE management and Client Conducting and assisting with event and incident investigations

Disseminating safety bulletins on workplace events and incidents

Self-auditing of the Project Safety Management System

All training undertaken on site shall be sent to the Training Department  training@scee.com.au to be uploaded in to MyPass.

Conducting worker and supervisor inductions

Comply with the requirements of the RTIO Critical Risk Management process

Assisting supervisors with conducting risk assessments

Conducting daily site inspections and weekly documented site inspections

Managing hazardous materials on site

Management of workers compensation and rehabilitation on site, in conjunction with head office support

Attending project safety, and Committee meetings, with the findings being reported to the necessary personnel

Preparing and presenting safety presentations at weekly toolbox meetings

Monitoring and reporting on safety initiative programs

Reviewing the weekly HSE performance (lag and lead indicators) with the Project Manager

Undertaking daily informal field inspections and documenting any hazards identified

Conducting, preparing and assisting with risk assessments as required

Managing SCEE’s project HSE requirements

Assisting in the review and updating of Work Instructions, SWMS and JHA’s as required

Reviewing all permit systems prior to authorisation from the Client

Inspecting First Aid facilities and replenish stock as required

Immediately stopping any “Unsafe Behaviour” identified during daily work activities

Conducting formal JHA reviews

Implementing SCEE and Client HSE Management Systems

Refer:	RTIO-HSE-0075146 Organisational Resources, Accountabilities and Responsibilities

### Construction Manager/Supervisors

Supervisors are to coordinate and direct all project works in their area of responsibility.

Supervisors report to the Project Manager / Construction Manager and are responsible for:

Assisting the Project Manager/ Construction Manager and the HSE Advisor in achieving the project HSE requirements

Understanding the requirements of this HSEMP

Planning to do all work safely via compliance to the applicable procedures.

Promoting HSE awareness at every opportunity

Identifying HSE training needs and informing management of these needs

Ensuring personnel are suitably skilled to undertake their assigned work tasks

Promoting and conducting hazard identification and reporting

Assisting in the preparation of JHA’s/SWMSs with workers

Conducting HSE inspections and initiating rectification of any issues

Participating in event and incident investigations

Facilitating pre-start and toolbox meetings (which may be prepared by the HSE Advisor)

Mentoring new starters or workers who have returned to site from leave

All training undertaken on site shall be sent to the Training Department  training@scee.com.au to be uploaded in to MyPass.

Monitoring subcontractor health and safety performance

Ensuring that adequate lighting is provided and maintained at all workplaces

Ensuring there is clear demarcation of all access ways, walkways, storage areas and roads

Ensuring machine guarding is in place, adequate, and maintained

Ensuring that all hard barricading is adequate, maintained and installed around penetrations as per site requirements

Comply with the requirements of the RTIO Critical Risk Management process

Ensuring that all workers who work at height comply with relevant Work Instructions and procedures

Participating in event and incident investigations

Undertaking weekly formal inspections of work areas Undertaking daily inspections of workplaces under their control

- Promoting environmental awareness at every opportunity

- Identifying environmental training needs

Reviewing all JHA’s /SWMSs in the field daily

Ensuring all relevant permits are in place prior to commencement of work

Ensuring a high standard of housekeeping is maintained at all times

Removing or isolating any hazard identified during daily work activities

Immediately stopping any “Unsafe Behaviour” identified during daily work activities

Assisting and supporting the HSE Advisor with the resolution of safety issues when they arise

Supervisors will be engaged in ongoing observations of worker teams to facilitate the promotion of sound “safe behaviours” and to correct “at risk” behaviours.

Refer: 	RTIO-HSE-0075146 Organisational Resources, Accountabilities and Responsibilities

### Health and Safety Representatives

Project Health and Safety Representatives, where elected take an active and participative role in such things as workplace inspections, either as an individual or in collaboration with SCEE /or Client HSE personnel. Elected Health and Safety Representatives will participate in applicable training courses sourced from a Registered Training Organisation in order to ensure they have the necessary skills to fulfil the position.

Where elected, the Health and Safety Representative is responsible for:

Assisting the Project Manager / Project Supervisor and the HSE Advisors in achieving the project safety requirements

Undertaking duties assigned under applicable legislation

Consulting with Project Management on all safety matters

Participating in safety meetings and programs

Always Promoting a high level of safety awareness by providing information to workers regarding project safety

Reporting to management any safety matters that are of concern by workers

Immediately stopping any “Unsafe Behaviour” identified during daily work activities

Assisting and supporting the HSE Advisor with the resolution of safety issues when they arise

Attending project HSE committee meetings

Health and Safety Representatives will be appointed as per applicable legislation.

# Personnel Management

## Worker and Sub-Contractor Selection and Mobilisation

When selecting candidates for employment on the project, SCEE shall ensure personnel hold the appropriate current licenses or certificates necessary to perform their duties. SCEE shall also check verification of competency (VOC) validity and organise for further training or VOC’s were deemed necessary.

SCEE shall adhere to the Client procedures for site entry, including completion of necessary site entry and accommodation request forms, pre-employment medicals and drug and alcohol testing. Medical assessments are carried out by Sonic Health Plus unless an alternate provider is specified by the Client.

SCEE will submit to Client representative’s details of prospective workers and sub-contractors for approval prior to mobilisation to site.

SCEE shall ensure any new site workers or subcontractors complete the following prior to commencing work on the project:

SCEE Company Induction

RTIO Site Inductions (Client)

Site Specific Induction

Construction Safety Awareness Training (White Card)

Medicals – Pre Employment, Drug and Alcohol

SCEE standard VOC’s and any Project specific VOC’s applicable to the relevant qualification and role

All workers will be provided with a position description outlining their roles and responsibilities.

## Pre-Employment Medicals

As a condition of employment, all site personnel are required to abide with Fitness for Work (FFW) procedures. This will commence with a pre-employment medical examination and a drug and alcohol screen prior to employment.

Prior to mobilisation, all site personnel will undergo a comprehensive Pre-employment Functional Assessment (PEFA), to assess their physical ability to perform the required tasks of their role. The PEFA assessments are customised for the physical demands of specific job profiles and assess the musculoskeletal risks associated with the particular task requirements for individual job roles.

All pre-employment medicals, PEFA’s, and drug and alcohol screening are coordinated by the SCEE Human Resources Department.

## Induction, Training and Competency

SCEE shall ensure that all site personnel hold current and relevant qualifications for the work they are required to perform and that records are always maintained and available. SCEE shall organise any competency assessments as required.

Assessments and supporting documentation verifying the competencies of workers shall be in accordance with Australian Standards, legislation and Nationally Accredited Standards as required. Training providers for registered training packages will be appropriately qualified and sourced from a Registered Training Organisation (RTO).

All training undertaken on site shall be sent to the Training Department  training@scee.com.au to be uploaded in to MyPass for inclusion into the project training matrix.

SCEE shall develop and maintain a Training Matrix through MyPass for all workers on site.

- Refer:	SCEE-TR-TD-PRO-0001 Training Procedure

- RTIO-HSE-0084278 Training, Competency and Awareness Work Practice

## 4WD Training

Any person required to drive a 4WD light vehicle on a formed unsealed road (i.e. rail access road which has been graded and regularly maintained) is required to complete the one day “Operate a light vehicle” RIIVEH201A/B” course or equivalent.  If a person is required to drive a 4WD light vehicle on a non-formed unsealed road (i.e. bush track) or drive in remote areas in steep terrain, they are required to complete the two day “Operate and maintain a 4WD - RIIVEH305A” or equivalent.

## SCEE Site Specific Induction

Prior to commencing work in the field all SCEE personnel shall attend a site specific induction.  The SCEE HSE Advisor, or designated person, shall provide the induction and the induction shall be documented and recorded on the project training matrix.

SCEE Project Managers shall ensure that the SCEE induction compliments the Client induction and that it is revised as required, to include any new policies and procedures throughout the duration of the project. The induction shall include a review of the Client and SCEE Safety and Environmental Management Plans as applicable to the SCEE scope of work.

As part of the induction process all workers shall be made aware of their responsibilities in accordance with this HSEMP and the Client Health and Safety Management Plan.

Project Management is responsible for ensuring that all personnel, regardless of previous experience, attend and participate in the SCEE Project Specific Induction.

All personnel completing the SCEE Project Orientation Record shall be required to complete the Medical Advice Form during the induction and review their New Starter Skills Checklist with their direct supervisor prior to commencing work in the field, as part of the SCEE First Swing Program.

Refer:	SCEE-HR-RE-TEM-0008 Medical and Allergy Disclosure Form

SCEE-BS-HS-PRO-0012 First Swing Orientation Program

SCEE-BS-HS-LST-0008 New Starter Skills Checklist

RTIO-HSE-0084278 Training, Competency and Awareness Work Practice

## Area Specific Inductions

Personnel required to work in areas they are not familiar with must complete an “Area Specific Induction” that describes any specific HSE requirements or information pertaining to the work to be completed.

This Induction shall be provided by an approved supervisor allocated to that area.

## MSIC Identification Cards

Any worker who is required to access secure maritime/wharf areas within the project will require Maritime Security Identification Card.  SCEE’s mobilisation team will assist workers in completing their application for their MSIC clearance.  The current timeline for application to card issue is 5 weeks.

## E-Learning Portal

A web site https://riotinto.ls.poweredbyonsite.com/ is where workers can complete mandatory RTIO refresher training etc, this site can be accessed from any computer that has internet access.

On successful completion of the E-Learning training course a certificate of completion is provided which needs to be forwarded to the SCEE training department to ensure the qualifications are uploaded in Stems.

## SCEE First Swing Orientation Program

The initial swing for staff new to a project has been identified as a period of elevated risk and has been associated with increased unwanted events and injuries. The SCEE First Swing Program is designed to ensure new and new to site personnel are aware of the specific site requirements and encourages supervision and new starters to actively engage in a communication process that aims to match initial task assignment to the new starters experience and qualifications, along with providing an opportunity to raise issues that the new starter may have encountered and improve the mobilisation process for future workers.

Toward the end of the first swing, a follow up meeting shall be scheduled with the supervisor, where the new starter and mentor should review and clarify any issues that may have arisen during the first swing period. Following this session, the first swing survey shall be completed by the new starter and returned to site administration for processing.

New starters shall also participate in the RTIO Management Safety Brief (MSB) prior to commencing on site.

Refer:   SCEE-BS-HS-PRO-0012 First Swing Orientation Program

SCEE-BS-HS-LST-0008 New Starter Skills Checklist

SCEE-BS-HS-TEM-0030 New Starter First Swing Review

## Visitor Induction and Controls

Project Management shall ensure all visitors to SCEE receive a Visitors Induction, which shall consist of instruction on the specific safety requirements of the project. All visitors shall remain in the company of the person they intend to visit and be supervised at all times.

Refer:	SCEE-BS-HS-PRO-0008 Induction - Workers and Visitors

## Fitness for Work (FFW)

On site FFW will continue to be addressed by means of:

SCEE Induction FFW education and awareness

Blanket BAC daily testing of all personnel

Drug and alcohol screening on a random basis or where there is just cause

Fatigue management and education

Hydration management and education

Nutrition and lifestyle management and education

Stress management and education

Worker Assistance Programs (EAP)

SCEE will incorporate ongoing Drug and Alcohol testing throughout the duration of the project to support the FFW Site requirements.

SCEE Management and HSE Advisors will focus on these issues throughout the duration of the project via prestart meetings, safety newsletters, toolbox meetings, posters, notices and information sessions in conjunction with the Client.

All breaches of Fitness for Work are to be actioned with reference to the SCEE Fitness for Work Policy and reported to the SCEE HR Manager for action.

SCEE shall conduct a “for cause test” for alcohol and/or drugs following an incident which has occurred either in the workplace or at the accommodation villages. SCEE will endeavour at times to undertake this testing however may authorise an external party to assist as required.  RTIO will conduct for cause testing following any incident on site.  Requirements for testing will be under direction of the RTIO Project Advisor.

Where testing is not achievable by either SCEE or an external party, site is to contact the HR Manager for further assistance.

SCEE will work with the Client’s procedures with regards to Pre-employment Medicals, Fitness for Work and Fatigue Management.

Refer:	SCEE-BS-HS-POL-0003 Fitness for Work Policy

SCEE-BS-HS-PRO-0025 Fatigue Management

SCEE-BS-HS-GUI-0001 Event Notification Guide

## HSE Discipline

SCEE Project Management shall ensure all personnel are aware that any breach of the SCEE Code of Conduct or repeat or deliberate non-compliance with SCEE or Client HSE systems or requirements may result in disciplinary action. In the event that disciplinary action is required then SCEE performance management and disciplinary procedures will be implemented with reference to Client HSE Procedures.

The Human Resources department shall be contacted for all disciplinary proceedings and will coordinate the process.

Personnel will also be advised that there may be times where a single incident may involve multiple levels of disciplinary action or removal from site. Some instances may include but not be limited to breaches in isolation and tagging procedures, alcohol, and drug abuse at the work site, tampering with or damaging safety equipment.

Refer: 	SCEE-HR-HR-POL-0003 Code of Conduct Policy

SCEE-HS-HS-PRO-0003 Code of Conduct

## Verification of Competency and Re-Training Frequency

SCEE undertakes Verification of Competency (VOC) testing of all operators of plant including EWP’s, Forklifts, Telehandlers, Cranes, Backhoes and Excavators and identified high risk work activities.

VOC’s and retraining/refreshers are undertaken in selected courses according to the following guidelines:

| Competency | Frequency (Years) |
| --- | --- |
| Blue / White Card | One off |
| VOC’s (EWP, Forklift/Telehandler, Crane, Backhoe, Excavator) | 2 |
| CPR (Electrician) | 1 |
| First Aid | 3 |
| Confined Space | 2 |
| Driver Training | 3 |
| Safe Working At Heights | 2 |
| Portable Fire | 2 |

# Consultation and Communication

SCEE Project Management will schedule regular structured meetings to ensure effective communication, consultation, and participation by all levels of the workforce.

Refer:	SCEE-BS-HS-PRO-0028 Consultation and Communication Procedure

## Pre-Start Meetings

SCEE Supervisors shall conduct a prestart meeting with all workers (including subcontractors) under their control prior to the commencement of work or when personnel are transferred to a new location or task. Pre-Start’s shall be undertaken utilising the RTIO CRM Sustaining Capital Pre-Start board as established by SCEE.  The prestart should address as a minimum:

Daily FFW / BAC Testing

The day’s activities

Any notices, bulletins, hazards, event or incident findings

HSE matters from adjacent work groups or contractors

HSE matters relevant to the task

JHA/SWMS and WI’s relevant to the task

Any permit requirements relevant to the task

Any changes in the status of the work environment

All safety precautions to be taken

Due to the location of the accommodation from site, personnel who are sick and staying in camp accommodation shall be visited prior to departure from camp and the RTIO CMS rep advised of a potential later start. Those workers who stay at camp will then be dealt with on a case-by-case basis but as a minimum contact will be made with two hours of departure from camp to check their wellbeing and ask if any assistance can be provided (e.g., site medic or doctor’s appointment required).

At no time shall a person who lives in camp accommodation and who does not turn up for work or contact their Supervisor advising of their absence from work, remain unvisited throughout the working day.

Late comers shall sign on a separate sign-on sheet in the SCEE site office. Their immediate supervisor will be informed of their arrival and make arrangements to transport them to the work front and receive the prestart information.

Refer:	SCEE-BS-HS-PRO-0028 Consultation and Communication Procedure

SCEE-BS-HS-TEM-0024 Pre-Start Safety Talk Checklist

SCEE-HR-HR-TEM-0048 BAC Test Register

## Refocus Meeting

In addition to the Pre-start Meeting workers who have return from their roster break or leave shall undertake a Refocus Meeting prior to commencing work.

The Refocus Meeting shall cover the following items as a minimum;

- Review current work fronts and any changes to scope. Focus should be on Management of Change.

  - Review past three days site notices

  - Review of any events

  - Review of any Procedure changes/requirements

  - Review open HazOb’s

  - Fatigue Management

Refer:	SCEE-BS-HS-TEM-0066 Refocus Meeting

## Toolbox Meetings

Toolbox meetings are the forum for workers to raise and discuss HSE topics. These meetings will be conducted weekly to present safety training talks and/or videos as organised by the Project HSE Advisor, Safety Committee, HSE Department or client as required.

Toolbox agenda items may contain:

Safety concerns raised, during the shift or week

Discussion of any unresolved safety issues

Discussion on safety aspects of planned work

Discussion of any changes to safety procedures

A specific HSE topic

Review of events or incidents during the week & the corrective action taken or planned

Discussion on results of audits and inspections

Risk Register contents

HSE initiatives and performance

Minutes of meetings are to be recorded on the Toolbox Meeting Report Form by the SCEE Site Manager/Supervisor and distributed for action as required.

Copies of the minutes will be posted on notice boards and filed for record.  All Toolbox Meeting Minutes are to be submitted to the SCEE Head Office within 24 hours of occurrence, complete with attendance sign on sheets.

Refer:	SCEE-BS-HS-WIN-0011 Safety Toolbox Meetings

SCEE-BS-HS-TEM-0025 Toolbox Meeting Report Form

## HSE Promotion

Project Management shall decide upon and implement HSE promotional activities including:

HSE Awareness Campaigns

HSE Posters and Talks

Reward and Recognition Programs

- Project Management shall be proactive in all site safety promotional schemes.

## Project Safety Committee

A project Safety Committee Meeting will be held monthly as a minimum, dependent on the type of work, number of workers and changing shift patterns.

The Committee will operate in accordance with the requirements of the relevant legislation and either the SCEE Site Manager or HSE Advisor will chair the Committee. The Committee shall be comprised of:

- SCEE Site Manager

- SCEE HSE Advisor

- Elected Health and Safety Representatives

- Nominated worker representatives from work areas/groups

At least half of the committee will be worker representatives. Subcontractor representatives may also be requested to attend the Safety Committee Meetings. Supervisors may be required to attend meetings when matters relating to their work areas are to be discussed. The purpose of the Safety Committee meetings is to discuss safety, health and environmental issues relevant to the project and have a priority on raising HSE Awareness on site.   The Safety Committee Meeting minutes will be recorded and made available to all personnel on project noticeboards.

Due to the size of the workforce on some projects forming a SCEE project health and safety committee may not be practicable, in such cases a health and safety committee may be run jointly with other contractors or SCEE may use other processes such as toolbox meetings to convey HSE information.

Refer:	SCEE-BS-HS-WIN-0013 Project Safety Committee Meetings

RTIO-HSE-0072106 Communication and Consultation Work Practice

## Management Meetings

Safety must be included in all management meetings and shall be the first item on the agenda to ensure it is given the necessary attention.

Safety topics for management meeting could include:

Project statistics to date

Review of incidents indicating any trends

Identification of training needs or any changes to the Safety and Health Management System

Review of any ongoing alternative duties

Planning of work skills and safety training program

## HSE Notice Boards

A Dedicated HSE notice boards will be located in the crib room and include information on:

Minutes of Meetings

HSE Promotional Material

Emergency Response procedures, contacts and phone numbers

First Aid personnel and contact details

Fire Warden personnel and contact details

Relevant and current HSE alerts and bulletins

Any Safety related Notice board should be kept neat, tidy and up to date to ensure efficient communication of HSE issues and information.

## Safety Incentives

SCEE will implement a reward and recognition program that focuses on the following principles, and all personnel will be encouraged to:

Actively solve problems

Provide ideas and suggestions for improvement

Display positive behaviours and attitude toward HSE, their peers and others when at work and conducting business.

The program will acknowledge achievements by individuals or groups. SCEE will also recognise and reward individual achievers and provide them the opportunity to share their experience with the rest of the workforce.

Depending on the projects HSE performance and milestones (e.g. Man-hours LTI free) the site may be eligible for a safety award or incentive. This eligibility is based on application and approval from Site and Project management.

Refer:	SCEE-BS-HS-PRO-0007 Safety Reward and Recognition

## HSE Alerts and Information

The SCEE Safety Awareness Communication System (SACS) is the primary method by which information affecting worker safety is disseminated to all SCEE workers and work locations.

SACS is a three tier notification system. HSE Alerts and Bulletins are used to communicate and raise awareness regarding any significant HSE issues which may pose a risk to personnel, plant and equipment, environment or the company. These issues may be identified as a result of incident investigations, hazard analysis, inspections, audits or management system reviews.

HSE Newsletters are used to communicate positive HSE events throughout SCEE and to keep personnel informed of other activities of the business relevant from a HSE perspective

All SACS documents will be distributed electronically to all personnel and be posted on site HSE notice boards.

Refer:	SCEE-BS-HS-WIN-0010 Safety Awareness Communication System (SACS)

## Health and Safety Issue Resolution

When an issue relating to workplace health and safety arises, management and workers must in the first instance attempt to resolve the matter by consultation between themselves. Where personnel believe that an unsafe or hazardous situation exists, or that planned work constitutes unsafe work practices, the worker has a right and obligation to:

- Stop work and remove themselves from the perceived hazard or situation.

- Immediately advise other workers who are in the affected area or directly involved in the work where the hazard or danger exists.

- Immediately notify their Supervisor of the unsafe or hazardous situation.

Most safety and health issues will be resolved between the supervisor and the worker however, where a satisfactory outcome cannot be reached the HSE issue resolution process will be initiated in accordance with the SCEE Health and Safety Issue Resolution procedures.

All managers, supervisors and workers shall be made aware of the SCEE consultation and issue resolution procedures and how they shall be applied. SCEE Site Management will also comply with Client requirements for Reporting and Resolving Health and Safety Issues.

Refer:	SCEE-BS-HS-WIN-0012 Health and Safety Issue Resolution

## Signage

The Project Manager and HSE Advisors have the responsibility to determine the relevant HSE signage to be displayed at appropriate visible points across work sites and locations. Signage may be permanent or temporary and shall comply with all site requirements and AS 1319. Signs shall be posted to assist in the prevention of events and help identify hazards.  Signs shall also be posted to indicate the location of safety equipment and devices such as fire extinguishers and first aid kits.

Signs shall be placed so that they can be readily seen and should be maintained in a clean and readable condition.

# Hazard and Risk Management

## General

It is a requirement that all SCEE operational sites and office locations implement HSE hazard and risk management processes across their scope of operations, and those processes are applied to all activities that SCEE can control or influence.

HSE hazards and risks must be assessed, prioritised and managed as appropriate to the nature, scale and potential impacts of operations and activities both individually and as a whole.

The hazard and risk management process shall be applied to all aspects of the Project including pre-mobilisation planning, mobilisation, project execution and project close out/review.

Refer:	SCEE-BS-RM-PRO-0002 Operational Risk

## Hazard Identification

- SCEE Project Management and Supervisors shall ensure hazard identification, reporting and control is implemented across all aspects of the Project. All personnel are required to participate in hazard reporting as per site and Client requirements, with the understanding that under legislation worker’s (including subcontractors) are required to report any situation in the workplace which they believe could constitute a hazard to any person.  This has now been deemed a KPI by the SCEE board and will be looked at running in conjunction with the clients CRM.

Refer:	SCEE-BS-RM-PRO-0002 Operational Risk

RTIO-HSE-0062207 Hazard Identification and Risk Management Work Practice

RTIO-HSE-0065318 Level 1: Job Hazard Analysis Work Practice

RTIO-HSE-0062204 Level 2: Risk Assessment Work Practice

## Hazard Observation Reports (HazOb)

The SCEE Hazard Observation (HazOb) report books are made available to all personnel on site at all times, or the Client HazOb cards may be used, where required. All hazards are to be reported even those that were rectified immediately by the person who identified it. This reporting process allows the information to be used to identify trends and assist other work groups or project sites to improve safety on the job.

Hazards reported will be recorded in the hazard observation register for the project and all corrective actions allocated logged in STEMS. All reported hazards shall be followed up, closed out and feedback provided to the work group in a timely manner.

Refer:	SCEE-BS-HS-TEM-0012 Hazard Observation (HAZOB)

SCEE-BS-QU-PRO-0007 Non-Conformance (Including Corrective and Preventative Actions)

RTIO-HSE-0062207 Hazard Identification and Risk Management Work Practice

## Hazard and Risk Assessment

All hazards and risks shall be assessed by applying the SCEE Risk Matrix.  Risks are assessed using a combination of Likelihood and Consequence.

When assessing risk controls, consideration shall be given to both preventive controls (to prevent an incident/ event occurring) and mitigation controls (to minimise the consequences should the incident/ event occur).

The approved L2RA content will be utilised to assist in the develop of risk assessments.  JHA risk assessments will be only valid for one shift. SCEE shall follow Pre-task hazard assessments flow chart - RTIO-HSE-0334261 when addressing risk.

Refer:	RTIO Risk Matrix

RTIO-HSE-0334261 Pre-task hazard assessments flow chart

## Risk Control

For every Hazard identified at least one control measure must be implemented. All controls shall be developed in accordance with the Hierarchy of Controls:

Elimination -  controls involve eliminating either the substance or the activity which gives rise to the risk and is the most effective form of risk reduction;

Substitution - controls involve substituting high risk products or activities with alternative lower risk products or activities to reduce overall risk exposures;

Isolation - controls involve isolating a hazard and may be achieved by distance, the use of effective barriers or a combination of both;

Engineering - controls involve making engineering changes to a process or piece of equipment used to perform a task;

Administration - controls consist of properly designed and implemented work practices and procedures;

Personal Protective Equipment (PPE) - is considered the last line of defence against hazardous substances or conditions.

## Work Instructions

SCEE has developed Work Instructions defining the required controls for tasks which are performed on a regular basis and work activities that SCEE has identified as being high risk.  The Work Instructions specify the method of work to be followed when conducting the task.

Work instruction information is communicated to each worker through toolbox sessions and made available to all personnel when performing identified activities.

Refer:	SCEE-BS-RM-PRO-0002 Operational Risk Management

## Safe Work Method Statements (SWMS)

Safe Work Method Statements (SWMS) shall be developed for all identified high risk activities identified in the Level 2 Risk Assessment.

Initially the task is run on a JHA . SWMS require RTIO senior advisor approval prior to being implemented.

SWMS must be reviewed by the workers involved and specifically apply to the high risk activity and the actual work environment.

Any tasks with an identified extreme residual risk are not to proceed and require referral to the Project Manager and HSE Supervisor for assessment. Divisional General Manager (or higher) approval is required for the activity to commence.

SCEE shall follow RTIO’s Pre-task hazard assessments flow chart - RTIO-HSE-0334261  & Iron Ore Standard Work Procedure –  RTIO-HSE-0067651

Refer:	SCEE-BS-HS-PRO-0002 SWMS Development

RTIO-HSE-0067651 Iron Ore Standard Work Procedure

RTIO-HSE-0334261 Pre-task hazard assessments flow chart

## Take 5

Completion of a Take 5 Risk Assessment (Take 5) is required for all tasks. It allows hazards and level of risk to be assessed prior to work commencing and also serves as a final check to ensure controls from other processes such as JHA’s and formal risk assessments have been implemented.

The Take 5 is a personal real time risk assessment tool designed to assess a person’s individual exposure to risk when performing tasks, and as such should be seen as necessary in the following situations:

When commencing any new task

When changes to the workplace occur, such as inclement weather or task modifications

When commencing involvement in tasks controlled by JHA or SWMS

When changing the method of work (e.g. removing PPE)

Refer:	SCEE-BS-HS-WIN-0002 Take 5 Risk Assessment

## Job Hazard Analysis (JHA)

A JHA must be developed by work crews for all activities and tasks that have an identified risk rating of medium or above that are not being conducted under a SWMS.  Tasks are to be analysed in accordance with the following criteria:

What are the steps making up the job or task? (Steps)

What are the hazards that could impact on each step of the job? (Risks)

What action is to be taken to remove or reduce the hazard? (Controls)

The Supervisor is responsible for identifying tasks for analysis and for conducting the JHA process on a daily basis with the work crew. The JHA may complement an existing Work Instruction and in such a case the relevant WI will be attached to the JHA present at the work front. The JHA will be developed on the RTIO JHA template.

SCEE Project Management shall ensure:

JHA’s are developed with the participation of all workers involved in the task

A JHA will be initiated prior to commencing tasks

All workers understand the requirements and have signed on to the JHA

JHA’s are reviewed and signed off by Supervision daily

The information from common task JHA’s (e.g. cable pulling, terminating) become SCEE WI’s  for that task

A register is maintained for all JHA’s

Records of all JHA’s shall be archived when the job has closed

Where WI’s exists, they are appropriately communicated to worker’s

The approved L2RA content will be utilised to assist in the develop of risk assessments.  JHA risk assessments will be only valid for one shift.  SCEE shall follow Pre-task hazard assessments flow chart RTIO-HSE-0334261 when addressing risk.  All identified changes will be communicated to the work crew and the amended JHA signed off by the Supervisor.

| Job Step List the tasks required to perform the activity in the order they are carried out | Potential Hazards What can kill or harm you in this job step (refer to section 2 & 5) | Agreed Controls What are the controls that will prevent you from being harmed or killed? | Accountable Person Who is the person responsible for ensuring the controls are in place and effective |
| --- | --- | --- | --- |
| Pre-Start Vehicle | - Unwanted movement - Engine Start up - Pinch Points - Hot engine parts - Oils & fuel - Vehicle faults | - Chock wheels - Isolate Vehicle and remove key - Keep hand away from any potential line of fire hazards - Wear gloves that protect from hands from heat, oils & fuel – review SDS. - Record all faults in prestart book and report to supervisor | Vehicle operator / driver |
| Drive vehicle | - Other traffic - Road works - Pedestrians - Weather - Fatigue - Vehicle faults | - Obey road signs and give way - Slow down when passing road works and stop when instructed - Slow down and give way to pedestrians - Check weather forecast before start of journey - Report any fatigue related issue to supervisor. Take a 20 min break every two hours of more if required - Complete journey management form when travelling more than 50 kms from site - Should any issues / faults arise with the vehicle, STOP and investigate and report to supervisor | Vehicle operator / driver |

What looks good:

What looks bad:

| Job Step List the tasks required to perform the activity in the order they are carried out | Potential Hazards What can kill or harm you in this job step (refer to section 2 & 5) | Agreed Controls What are the controls that will prevent you from being harmed or killed? | Accountable Person Who is the person responsible for ensuring the controls are in place and effective |
| --- | --- | --- | --- |
| Pre-Start Vehicle | - Unwanted movement - Pinch Points | - Isolate Vehicle - Wear correct PPE | All |
| Drive vehicle | - Other traffic - Vehicle faults | - Obey road signs - Report to supervisor | All |

Why this looks bad – How do we isolate the vehicle, what is the correct PPE for the task, What are we reporting to the supervisor and why is everyone accountable when the driver/operator is in charge of the vehicle.

JHA’s shall be detailed and clear at all times so anyone new coming into the work area understands the task and controls in place.  Being short or using abbreviated terms is not acceptable.  For example, the task is to splice a cable using a sharp tool.  We need to use cut proof gloves to undertake this task, so the JHA should not read “use correct PPE” it should read “Ensure Cut Proof Rated gloves (Minimum Cat 5) to be always used to avoid cuts or lacerations.  This way it is in more detail and the person undertaking the job knows exactly what is required and why the control is in place.

Refer:	SCEE-BS-HS-PRO-0001 Job Hazard Analysis

SCEE-BS-HS-TEM-0008 JHA Form

RTIO-HSE-0065318 Level 1: Job Hazard Analysis Work Practice

RTIO-HSE-0334261 Pre-task hazard assessments flow chart

# Control of Hazards / Risks

## 5 Star Commitment

SCEE has identified a series of significant fatal risks to its workers and subcontractors.  These risks are:

Electrical Isolations

Management of Change

Lifting Operations

Working at Heights

Driving Safety

The 5 Star Commitments do not cover all operational risk, nor do they replace SCEE’s Risk Management Processes that shall be used by both workers and subcontractors.  They set a minimum standard for establishing a safe work environment, set clear expectations for safe behaviour, assist in creating a safety mindset and provide a checklist for ensuring safe behaviour of one’s self or others, regardless of where the activity is taking place in SCEE’s operations.

Refer:	SCEE-BS-HS-PRO-0027 5 Star Commitment Procedure

## Critical Risk Management (CRM)

SCEE shall follow and contribute to the RTIO CRM processes and ensure they are followed as outlined below.

Contractor’s personnel completing physical tasks or work on site are to be considered equivalent to ‘operators’ and/or ‘maintainers’ when using the CRM system.

CRM focuses on 22 risks that have the potential to kill workers at work and involves everyone in checking that lifesaving controls are in place and working.

CRM involves everyone playing their role in checking that the critical controls for every job are in place and working. If they’re not, the job STOPS.

Workers and supervisors discuss the risks and controls for a task and using a checklist, record that the control is in place and effective. The verifications performed by Leaders/Supervisors are loaded into the CRM online system. Workers fill out the paper-based checklists.

The following objectives have been categorically broken down to summarise the overall intent of the CRM program:

| System |  |
| --- | --- |
| Signage | All must be aware of the critical risks before getting to their work area, plus the use of visual reminders just before starting a critical job. |
| Integration in Visual Management | CRM is being actively managed every day at all levels of the organisation |
| Information technology & CRM Portal | Site selects the appropriate technology to support implementation and ensure it will always be available for key personnel |

| People |  |
| --- | --- |
| In field training/coaching | Site has an established leader coaching system (with HS support). All involved will be coached to ensure CRM process is properly applied and drives culture change. |
| Training & communication | CRM and critical risks/ controls are well understood by all, including new workers and Contractors. CRM is part or regular communication at site. |
| Culture: Stop & seek help | Leaders actively seek red as an opportunity. Workers feel empowered to delay starting a job until comfortable all controls are in place and effective (or containment ensuring their safety). |
| Level of penetration | Everyone on site is involved in CRM |

| Process |  |
| --- | --- |
| Quality & Actions | Verifications conducted are effective in assessing whether non‐compliant controls are identified and addressed. |
| Reports & Data Analysis | Reports are used to drive improvement. Above and beyond standard reports, portal data can reveal important information with analysis. |
| Integration | Well integrated in business processes. Different tool integration is captured and actioned, so the use of CRM and other tools is most efficient. |

In keeping with the CRM process, SCEE shall complete the following:

- Critical Control Checklists (CCCs)

These must be completed by the SCEE’s personnel after set-up, but before commencement of any works of any works involving the 22 identified critical risk areas. The checklist must be completed every time that work is conducted in one of these critical risk categories.

- Critical Control Field Verifications (CCFVs)

These must be completed by SCEE’s supervisor and will be completed by the RTIO Representative throughout the duration of the Works. This checklist reviews the controls that have been put in place.

- Critical Control Verification Standards (CCVSs)

These are completed by managers and superintendents within RTIO, to ensure that the processes and procedures ensure that works are completed safely.

Refer:	RTIO-HSE-0289260 CRM Critical Risks and Controls

## Family Star

At SCEE Electrical, leadership is considered crucial for improving performance and promoting health, safety, environment, and quality within the organisation. Effective HSEQ leadership serves as a key factor in reducing serious health and safety incidents as well as high potential events. To foster a strong team culture, it is essential for personnel to demonstrate HSE culture behaviours, exhibit curiosity, challenge norms, and genuinely care for others. This helps create a mature team environment that attracts individuals who want to be part of it.

By integrating these elements into the overall approach, SCEE Electrical aims to foster a strong safety culture, empower workers to be proactive in risk management, and continuously improve performance in all areas of the business. The focus on leadership, family-like care, drift awareness, work culture, critical control points, and cross checking contributes to creating a safe and thriving work environment where individuals are motivated to be their best and look out for one another.

The Family Star Initiative foundations are based on one question - Who has the least to say in how we go about our work each day and has the most to lose?

The Family Star Initiative is based around the following areas;

- Family Values

- Drift

- Group Space/Head Space/Work Space

- Critical Control Points

- Cross Checking

This initiative will be presented by Site Management as a face to face session with all workers and supported throughout the duration of the project.  The sessions will be recorded in Stems and form part of the Project Training Matrix.

## Fatality Prevention Program (FPP)

The purpose of the FPP is to identify potential major and catastrophic situations in the workplace that could lead to a fatality, or multiple fatalities.  These Potentially Fatal Events (PFE) are ‘high consequence, low probability’ situations, which are identified and controlled to reduce or eliminate the associated risk.

The aim of the FPP is to involve the entire Project leadership team (managers, supervisors and H&S representatives) from SCEE and the client who are experienced in mining/construction activities to help identify PFEs using a structured process and ensure that effective controls are in place to minimise the risk.

SCEE shall provide input to the Site FPP register which is maintained by the client.  The FPP register records PFEs identified and the status of corrective actions.

SCEE acknowledges and agrees that an essential part of the FPP is ensuring that all Personnel understand the energies that can cause them serious harm, and the critical controls required to be in place to reduce the risk of a fatality occurring.  SCEE’s Personnel shall participate in the ongoing monitoring of critical controls in the workplace (CRM Tools) which must be conducted with involvement of its supervisors and managers.

Site HSE forums will be held to bring together management, supervision and construction Personnel in a collaborative forum to address safety related topics and issues relevant to the Projects.  Team members will collectively develop agendas and training sessions relevant to the Site.  The Site HSE forums shall be attended by SCEE’s personnel including management, supervision and the workforce.

## Stop Work Authority

SCEE is committed to providing a safe work environment for all workers and contractors working on site. As such, it is a duty and the right of every worker and contractor at SCEE to exercise the Stop Work Authority whenever the worker feels at risk in completing a job/task. Management supports the decision of its workers in the diligent execution of the Stop Work Authority.

The following steps shall be followed when the Stop Work Authority has been initiated:

- An worker may cease or refuse to carry out work if the worker has a reasonable concern that his/her health, safety or wellbeing is at risk from an immediate or imminent exposure of a hazard.

- The worker instigating the Stop Work shall inform his/her Supervisor, HSE Advisor or safety representative (if appointed) immediately when work has stopped.

- The HSE Advisor or safety representative may direct an worker to cease work if the representative has reasonable concern that the job/task would expose the worker to a serious health and safety risk from exposure of a hazard. The Safety Representative or HSE Advisor shall inform his/her Supervisor immediately when work has stopped.

- It is the responsibility of the Supervisor to ensure that the hazard is isolated, and the Stop Work Authority is maintained until control measures have been implemented to reduce the risk to an acceptable level.

- If the hazardous situation cannot be rectified immediately the worker must be allocated alternative work in the same or another workplace if that work is safe and appropriate for the worker depending on the worker’s competency level.

- An worker’s employment and entitlements shall not be affected by invoking the Stop Work Authority if the worker has not unreasonably failed to comply with a direction to carry out suitable alternative work at the same or other workplace that was safe and appropriate for the worker.

## Mobile Phones

With the introduction of many approved applications (e.g., SharePoint, Critical Risk Management, Leadership in the field, Stems) the safe use of devices across sites is essential.

SCEE Electrical with the approval of the client will allow for a device to be used to access approved online systems while providing specific work area guidance to prevent personnel using devices in a way that may lead to an incident or distraction.

Mobile communication devices permitted in operational areas with specific guidance regarding use e.g.

- For work related activities on approved SCEE/Client applications.

- To take photos or capture video footage that will be used specifically for work purposes only. Once used the images shall be deleted.

- Earphones/pieces shall not be used at any time when undertaking work related activities.

- Storage location in Heavy Mobile Equipment (HME) is to be in a secure location / crib bag - outside of view.  Storage in an operators pocket is not acceptable due to the risk of driver distraction.

- No phones when driving - Do not operate cell phones (whether handheld or hands free) while operating a vehicle.  Pull over to a safe location and stop the vehicle.  Once stopped answer or make the required phone call.

- No phones when completing any High-Risk Works.  If you need to use your phone, come back to ground level and go to a safe location before answering or making a call.

To ensure no one is exposed to a hazard while using a mobile communication device, all personnel must always ensure they:

Are stationary whilst using a device, i.e.;

- Not walking around (unless within office / admin areas where walking while talking is permitted provided that is done free of hazards;

- Not climbing up or down stairs;

- Not conducting physical work;

- Are in a safe environment or location; positioned clear of any hazards;

- Are aware of their surroundings; and

- Comply with directions issued by their leaders.

## Work Environment and Housekeeping

Project Management shall ensure all SCEE working areas are maintained to a high standard.  The following housekeeping requirements shall be complied with:

Work areas shall be kept neat and tidy

Storage facilities must be provided in all areas, sufficient to cope with requirements

Nothing shall obstruct safety, first aid, and fire or ventilation equipment

Platforms, stairways and ladders shall be kept clear, tidy and in good repair

Amenities shall be kept clean and hygienic

Oils and grease on floors shall be cleaned up immediately to prevent slip hazards

All unnecessary items must be removed from the workplace daily

Waste disposal containers must be provided and regularly emptied

Housekeeping will be monitored in formal site inspections and informal inspections of the workplace shall be conducted regularly by Project Management to ensure high standards of housekeeping are maintained. Any identified hazards shall be rectified immediately.

## Personal Protective Equipment (PPE)

All personnel are required to comply with the wearing and use of all issued PPE. Project Management shall ensure all PPE and clothing requirements set by the client, legislation and applicable Standards are complied with.

The following is the minimum protective clothing requirements on site:

Long trousers with reflective stripes

Long sleeve shirts Hi-Vis with reflective stripes

Leather steel cap lace-up protective footwear

Safety glasses

Gloves applicable to the task, including Hi-Vis gloves as required (to be carried and available at all times)

Hard hat

The required PPE for all tasks must be identified in the risk assessment (JHA /Take 5 / SWMS). Project Management shall ensure appropriate PPE is available for tasks requiring further protection e.g. hearing protection, respirators, face shields etc.

Project Management shall ensure that workers are appropriately trained in the use, maintenance, storage, disposal and limitations of the equipment prior to use.

All issued PPE will comply with Australian Standards.

Refer: 	SCEE-BS-HS-PRO-0013 Personal Protective Equipment Procedure

SCEE-BS-HS-GUI-0004 SCEE Minimum PPE Matrix

SCEE-BS-HS-GUI-0005 SCEE Glove Matrix

### Personal Adornments

All personnel shall be advised during the SCEE induction of the personal adornment requirements, as per site specific procedures.

Refer: 	RTIO-HSE-0109318 Iron Ore Work Attire and Adornment Work Practice

## Working in Heat

Where location or conditions have been identified as having the potential to cause heat stress conditions then supervisors where practicable, shall consider and apply the following controls when organising work:

Schedule demanding activities for the cooler times of the day

Ensure sufficient drinking water is available at work locations

Ensure sunscreen is available at work location

Job rotation for personnel to provide relief periods

Shaded rest areas to rehydrate

Provision of additional PPE, such as hat brims, neck wraps, over hat protectors etc.

At induction and throughout the duration of the project, personnel shall be advised and educated on such things as:

Acclimatisation

Identifying the symptoms of heat stress

Importance of fluid intake / hydration

Impact of Health and Lifestyle factors

Control strategies and work routines

Prevention of heat related illness

- SCEE shall undertake hydration testing during the project with a minimum mandatory three (3) tests per shift to track hydration risks.

Refer:  	RTIO HSEQMS Performance Standard B6 Thermal Stress

RTIO-HSE-0128390 Thermal Stress Work Practice

## Hydration Testing

SCEE shall undertake Hydration testing on all workers during the project.  The testing procedure will involve the MX3 Pro Hydration Testing System or urine self-test strips.  Testing will be conducted by randomly selecting workers at prestart, to conduct the hydration test.  The worker will be requested to provide 3 tests during their shift.  Blanket testing will be conducted from time to time.

Daily results shall be recorded on the Hydration Register and retained by SCEE Electrical.  The register will be provided to the client as requested.

Results of testing are to be addressed in accordance with the Hydration Management protocol Flowchart.

Refer:	SCEE-BS-HS-TEM-0066 SCEE MX3 Hydration Management Protocol Flowchart

## Permits to Work

All work that will require the use of permits shall follow the Clients permit system and requirements.

SCEE cannot work until a General Permit to Work is in place. The Permit to Work is signed daily by the RTIO Project Advisor assigned to the project.

SCEE may be required to obtain permits and approvals prior to commencing the Works, including but not limited to the following:

  - Approvals Request

  - Clearing Permit

  - Excavation Permit

  - Penetration Permit

  - Hot Works Permit

  - Working at Heights

  - Confined Space

  - Vicinity Permit

  - HV Permit

  - HV Access Permit

  - General Permit to Work

SCEE shall return Permits to the RTIO Project Advisor on a daily basis, or as requested by the RTIO Project Advisor.

## Working at Heights

SCEE is committed to eliminating all fall potential from project works.  For each job where workers are required to work at height and there is a risk of falling from one level to another a risk assessment shall be conducted to identify the level of risk and controls to be implemented.

SCEE Project Management and supervisors are responsible to:

Develop and maintain a system to ensure that work shall not be undertaken at any height without there being adequate fall prevention or fall protection measures in place

Ensuring only those workers with current working at height qualifications are allowed to work at heights

That risk assessments (JHA/SWMS) are conducted to identify hazards and the appropriate control measures are implemented

Some common controls that should be considered are:

Temporary Work Platforms – EWP’s, Scaffolding

Edge Protection – guard rails

Individual Fall Arrest Systems - Safety harness with lanyard

Restraint Systems - Static lines

SCEE will work in compliance with the Clients Working at Heights Procedures and Permits.

Refer:	SCEE-BS-HS-PRO-0017 Working at Heights (High Risk)

RTIO HSEQMS Standard C4 Working at Heights

RTIO-HSE-0090983 Working at Heights Work Practice

### Falling Object Prevention and Encapsulation

A Job Hazard Analysis (JHA) shall be completed prior to commencement of all work above ground level.

The risk of falling objects shall be controlled through Elimination wherever possible.  Where this is not possible, controls from at least one of the following principles shall be applied for all work above ground level. Where work is conducted within 2 metres of an edge or void, controls from a minimum of two of these principles shall be applied, to ensure a secondary level of defence:

- Secure – all tools, equipment and material during use, transport and storage;

- Encapsulate – barriers around work areas to prevent uncontrolled items falling from one level to another;

- Exclude – prevent people from accessing below the work in case the above controls fail.

Effectiveness of controls shall be monitored through the use of SCEE’s 5 Star Inspection Form – Prevention Dropped Objects.  This inspection shall be completed prior to any working at heights commencing.

Stored items such as ladders, conduit, boards, and other lengthy items shall be stored horizontally and not leant against structure, handrails etc.

## Working from Platform Ladders

The use of platform ladders is permitted provided:

The ladder is appropriate to the task

The ladder is in good condition (no broken rungs, rails or footings etc.)

The ladder is on firm, stable and level ground (this includes grid mesh if the ladder footings are larger than the grid mesh squares)

The ladder is the correct height for the task to avoid reaching or stretching

The body must be centred between the rails at all times

The ladder is secured against displacement (i.e. slipping or sliding) and/or there is another person holding the base of the ladder

All the locking devices on the ladder are secure

The ladder is always faced while climbing up or down and while positioned on the platform

The ladder complies with AS1657 and AS1892

The ladder has been inspected and tagged accordingly

Ladders must be fitted with fully enclosed railings to prevent people from falling off the platform.

All work from portable ladders should be minimised and use alternative safer means where possible.

Materials or tools are not carried by hand while climbing the ladder. Tools should be carried in a side pouch or handed up by an assistant.

The work undertaken while on the ladder should allow for three points of contact to be maintained and tools can be operated safely with one hand.

Refer:	SCEE-BS-HS-WIN-0020 Portable Ladders

RTIO HSEQMS Standard C4 Working at Heights

RTIO-HSE-0090983 Working at Heights Work Practice

RTIO-HSE-0049639 Iron Ore - Ladders, Stairs, Walkways, fixed and non-motorised work platforms Guidance Note

## Working Over/Near Water

SCEE is committed in ensuring workers are safe at all times when working over or near water.  Working “on, near and over” water is defined as working in close proximity to water where there is a reasonable likelihood of falling into the water. This is taken to mean less than 2m from an unguarded edge, above the water.

This risk management process shall include the creation of a risk strategy, identification of potential risks, and assessment of risk severity, development of risk response and monitoring of risk status. A Pre Task Risk assessment shall be performed on all work, to determine the most effective systems of control to enable personnel to perform work in a manner that they are not exposed to hazards. The risk assessment shall also consider all alternative control measures pertaining to working over or near a body of water.

When Working Over / Near Water; Working at Heights and Barricading procedures must be followed in conjunction with a task specific Pre Task Risk Assessment.

In addition to normal site PPE requirements the following shall be in place prior to commencing work.

- PFD 275N suitable for task – Hot Work Resistant if this work is to be undertaken

- 1 x life-ring with 25m of rope are provided on all berths

- 1x radio for contacting rescue vessel and / or EMO in case of emergency

- 1 x fully equipped rescue vessel Manned at all times by a minimum of 2 trained crew.

- Fall Arrest Harnesses (if required) may incorporate a PFD.

All PFD’s are required to be inspected and tested annually by an authorised organisation with certificates of inspection received and filed with tags in place.

A Drowning Critical Control Checklist shall be conducted prior to any works commencing.

Refer:	RTIO-HSE-0147039 Ports – Working Over / Near Water Procedure

## Excavations and Penetrations

All excavations on site will be carried out in accordance with Client Excavation, Penetration and Earth Work requirements.  All excavations and penetrations will comply with the referenced procedures and require the issue of a permit to excavate/penetrate, and the use of hard barricading as required.

All excavation or penetrations within 1.5m of high voltage, high pressure, hydrocarbon, optic fibre or communications services shall be hand excavated or pot holed. SCEE Project Management will ensure that where hand digging is to be conducted it will be with the use of shovels preferably. Picks and bars should not be used.  Where approved, an electrically powered Kango type hammer with a Spade bit can be used.

Where an worker is required to work in an excavation that is at least 1.5 metres in depth, the excavation or earthwork shall be shored, benched or battered in a manner which will prevent it from collapsing or moving.

SCEE Project Management and Supervisors will be familiar with and incorporate the Client excavation procedures and permits into their work procedures.

Refer:	SCEE-BS-HS-PRO-0021 Trenching and Excavating (High Risk)

SCEE-BS-HS-SWM-0018 Trenching & Excavation

## Isolation of Plant and Equipment

Project Management shall ensure that prior to workers being permitted to work on equipment or electrical systems that have a potential to become live, the Client’s isolation lock and tag-out procedures have been communicated to all SCEE workers via a formal training session.  All training shall be documented and recorded in the project training matrix.

A lock register will be maintained for all isolation locks issued to SCEE site personnel.

No personnel shall work on or near isolated plant and equipment unless they have applied a personal isolation lock and tag. A lock and tag will be applied to a scissor clip at an isolation point or to a permit lock box. Personnel shall sign onto the isolation permit if they are working under an isolation permit.

Light Vehicle Isolation requirements when performing a pre-start shall be for the individual completing the pre-start to have possession of the ignition key in their pocket.

SCEE Project Management shall ensure the Client’s isolation and lock and tag-out procedures are strictly complied with.

Refer:	RTIO-HSE-0049637 Isolation Work Practice

## Hot Work

Hot Works will include but is not limited to:

Grinding

Hot cutting

Electrical welding

Cad Welding

Any other process that produces or has the potential to cause a spark or ignition source

Where hot work is to be performed within a designated fire risk area or an area that is not normally a designated hot work area, a hot work permit shall be issued by authorised permit issuers.

SCEE shall have a trained fire watch at each hot works front for 30 minutes following the suspension or completion of hot works.

A fire extinguisher shall be available of hot work being undertaken. Where hot works are to be undertaken, the direct area should be clear of combustibles and welding screens used as required.  Gas testing shall be conducted prior to any hot work within an area with a suspected flammable atmosphere.

SCEE shall follow the procedure for Non-Permissible Hot Works Areas. Work areas that are flammable cannot have hot works until the RTIO Area Superintendent has signed off.

SCEE will comply with the guidelines of Department of Fire and Emergency Services (DFES) where “no hot works” will be allowed to be carried out as directed by this department on extreme or catastrophic days where total fire bans may be in place. SCEE may request an exemption, on a case by case basis, to continue with hot works (subject to additional fire watch conditions).

SCEE shall implement and follow the Sustaining Capital Grinder Checklist.

Refer:	SCEE-BS-HS-PRO-0015 Hot Work (High Risk)

SCEE-BS-HS-SWM-023 Welding

RTIO-HSE-0049671 Iron Ore (WA) Welding Work Practice

## Drilling into Cable Ladder and Panels

Any task that requires drilling into a cable ladder or a panel with cables installed will only be conducted under a permit.

This requirement applies no matter whether cables are live or de-energised if the ladder is full or has only one cable installed.

The permit will be signed by the nominated Project Supervisor and HSE Advisor.

Refer:	SCEE-BS-HS-TEM-0034 Drilling Near Cable Permit

## Machinery and Vehicle Inspection

All company machinery and vehicles shall as a minimum, be serviced in accordance with the manufacturers recommended intervals and in accordance with regulated requirements.

Additionally:

Project supervision shall ensure that suitably qualified personnel carry out regular maintenance

Records shall be maintained of all inspections carried out

MEWP’s and Cranes shall have documented annual inspections carried out

Site administration shall provide a copy of all service records to SCEE head office on a weekly basis

Inspections shall be carried out on all vehicles prior to mobilisation to site ensuring that the vehicle is free from weeds and seeds and that it is in sound serviceable condition

When performing maintenance, cleaning or repairs, the plant shall be stopped and isolated using the site lockout or isolation procedures. This may require the use of danger tags or permit to work systems.

SCEE Project Management will comply with the requirements of site procedures for Vehicles and Mobile Plant mobilisation and maintenance.

Site supervisors shall ensure all operators’ logbooks are completed on a daily basis and that operator manuals are available and in good condition.  Logbooks shall be retained in the vehicle at all times.

All defects of any machinery shall be logged in the Plant and Equipment Maintenance Register.

A detailed planned maintenance program for all equipment shall be maintained in Stems.

Refer:  	RTIO HSEQMS Machine Guarding Work Practice

RTIO HSEQMS Standard C1 Isolation

RTIO-HSE-0049637 Isolation Work Practice

RTIO-HSE-00049645 Vehicles and Driving Work Practice

## Driving

All driving must comply with the SCEE’s Journey Management Procedure SCEE-BS-HS-PRO-0022. Driving in particular must be managed to delay the onset of related fatigue, specifically:

- Plan all journeys; drive to road and weather conditions at all times but as a minimum standard obey the road rules

- Stop and rest for 15 minutes every 2 hours, with a minimum 30 minute break after 6 hours of driving

- Maximum driving time in any 24 hours is 10 hours with a minimum 10 hours rest afterwards

- Maximum 3 days driving 6 hours or more per day with a minimum of 1 rest day afterwards

- No driving if no sleep in the 16 hours prior to commencement of driving.

Drivers will only operate a vehicle when:

- They are medically fit to operate the class of vehicle being used.

- They assess the suitability and safety of the vehicle prior to any journey being undertaken.

- Have completed a verification of competency (VOC) where required

- They are not under the influence of alcohol (zero BAC reading) or drugs and are not suffering from fatigue or impairment.

- They do not operate cell phones (whether hand held or hands free) while operating the vehicle on a mine site.  Hands Free can be used offsite in accordance with WA Road Traffic rules.

- They have the applicable licence to drive the class of vehicle they are operating.

Passengers will only travel in a vehicle when:

- They assess the suitability and safety of the vehicle prior to commencing the journey.

- They assess the suitability of the drivers competency to undertake the journey

- The vehicle is fit for use – Keep it clean, report any damage.

Refer:	SCEE-BS-HS-PRO-0022 Journey Management Procedure

## Cranes, Lifting and Rigging Equipment

Prior to use on the Project, a competent person shall inspect all cranes, lifting equipment and complete a mobilisation inspection form for submission to the Client. This inspection report shall include:

Serviceability

Certification

Working Load Limits (WLL must be displayed at all times)

Component inspections (e.g. crane rope, block, etc.)

Compliance with statutory and site requirements

Cranes, mobile elevated work platforms and other lifting equipment shall be inspected daily and recorded in the daily inspection logbook

- Cranes shall have annual inspections conducted on them and copies of the inspection will be placed with the machine and in the SCEE site office. An entry pertaining to the next inspection date will be recorded in the plant and equipment maintenance register.

For this Project, a Critical Lift is described as any lift:

Lifts over operating facilities where this may endanger personnel;

Lifts over or near power lines or electrical transformers or substations (refer to vicinity permit guidelines for specific requirements);

Lifts involving personnel cages;

Lifts at or above 90% of maximum rated capacity of crane as configured for the task;

Multiple crane lifts;

Lifts with multiple hoists including load shifting devices (lever hoists / chain blocks) where the centre of gravity or the load share changes between the devices.

A critical lift requires close liaison with the Client and a documented lift study must be completed and submitted to the Client prior to conducting any critical lift.

All rigging and lifting work shall be performed in accordance with statutory requirements.  Only certified persons (rigger, dogman, crane driver) shall lift, rig up or supervise lifts.  Intermediate Rigger (RI) qualification to design or approve critical lifts and / or rig (reeve) cranes.  Lift analysis (JHA) shall be conducted for all heavy or difficult lifts.

- All lifting equipment used by the company shall be subject to regular inspections to ensure adequacy for use, this includes all slings, shackles and hooks etc. Cranes, mobile elevated work platforms and other lifting equipment shall be inspected daily and recorded in the inspection logbook.  Equipment inspections shall be performed by an authorised person who holds as a minimum a current Dogman’s qualification. Records of training shall be maintained for people performing equipment inspections. A register of all rigging equipment and test /

- inspection data is to be maintained by SCEE on site.

Inspections shall be carried out by a certified crane driver, dogman or riggers to gauge suitability for use and subsequently logged on the lifting equipment. All items shall be colour coded to verify inspection status.  The colour coding shall be in accordance with the electrical colour coding system.

Red 	-	December to February

Green	-	March to May

Blue	-	June to August

Yellow	-	September to November

Items to be checked for during the inspection shall include:

Any cuts, nicks, breaks and chafing

Wear or deterioration – stitching / chemicals

Elongation of chain / hooks

Identification number

Any item found which may affect the safe use of the equipment

Makeshift lifting equipment shall not be used under any circumstances.  Items found that do not meet a safe standard shall be withdrawn from use immediately, marked with a danger tag by the person carrying out the inspection and sent to head office for repair / disposal.

SCEE will work in compliance with the Client’s procedure for inspection, testing and tagging of equipment.

All lifting tasks shall be subject to a RTIO lift plan.

Refer:	SCEE-BS-HS-WIN-0026 Crane Activity

SCEE-BS-HS-SWM-0019 General Mobile Crane Work

RTIO HSE-0049767 C6-Cranes and Lifting Equipment

RTIO-HSE-0162237 Cranes and Lifting Work Practice

RTIO-HSE-0049645 Vehicles and Driving Work Practice

## Forklift/Telehandler Operations

Operators of forklifts on SCEE work sites shall have a Licence to Perform High Risk Work endorsed with load shifting equipment classification LF (Forklift Truck). Operators of telescopic handlers (Manitou) are required to have a certificate of competency for telescopic materials handler.

Operation of forklifts and Telehandlers shall be in accordance with SCEE Work Instructions.

Refer:	SCEE-BS-HS-WIN-0027 Forklift Telehandler

SCEE-BS-HS-WIN-0034 Telehandlers

RTIO-HSE-0049645 Vehicles and Driving Work Practice

## Loading and Unloading

When loading or unloading on site, the following shall be adhered to:

•	Inspect loads before un-securing to ensure it is safe and cannot not shift once binders are released.

•	Personnel are not permitted to stand on the offside of the truck during loading and unloading. This area is to be demarcated with cones and flags and personnel are to monitor that this area is clear prior to any touching or movement of loads.

•	No person/s is permitted to be on or climb onto an open tray without appropriate working at height protection; fall restraint protection such as gates or side rail protection will be a minimum of 900mm.

•	Use of over centre binders to secure loads are not permitted, ratchet type binders or webbing straps shall be used.

•	Chains and strapping should be protected from sharp edges.

•	Inspection of load restraint equipment should be conducted prior to use.

•	Unloading or loading in non-designated areas will be performed away from traffic flow to minimise interface issues.

•	Rolling equipment is to be adequately secured to prevent rolling during transport.

•	Dedicated lifting and tie down points of equipment are to be used.

•	Due to the inherent risks associated with lifting, moving or relocating transportable buildings, this task shall only be conducted using a crane. The use of a ‘Tilt-Tray’ vehicle is permitted only once a risk assessment has been undertaken.

•	All loading docks and ramps shall be engineered/ purpose-made and authorised for use.

•	Loading ramps should be assessed to ensure they are rated for the weight and positioned correctly for the width of the equipment.

•	Designated loading and unloading areas should be utilised where possible.

•	Spotter should be used as required.

•	Drivers shall remain clear of loading and unloading activities and to remain under the direction of the SCEE escort.

•	The off side during loading and unloading should be a no go zone for all personnel;

•	Non-inducted drivers to be escorted and informed of site requirements.

## Scaffolding and Static Lines

The erection and dismantling of scaffolding and static lines shall be carried out by a licensed scaffolder or under the direct supervision of a licensed scaffolder.  All static lines and scaffolds shall be inspected for damage on a daily basis. Scaffolds and static lines shall be registered on a Scaffold and/or Static Line Register.

Project Management shall ensure:

All scaffolds have handrails, mid rails, toe boards and access ladders

Minimum of Rigger (basic) to rig Safety nets and static lines

Only appropriately certified personnel will carry out scaffolding erection

Scaffolds are erected and dismantled using the Scafftag scaffolding tagging system and to site construction requirements.

The appropriate Scafftag insert is filled out and the Scafftag displayed on every scaffold whether under construction in use or being dismantled

All scaffolders shall be protected at all times from falls during the erection and dismantling of scaffolds

Aluminium scaffolds, ladders and harness clips shall not be used in MCC’s, substations or where caustic material is present.

Refer:	SCEE-BS-HS-WIN-0004 Scaffold Safety

SCEE-BS-HS-SWM-0013 Erecting/Dismantling Scaffold

SCEE-BS-HS-SWM-0014 Working on Scaffold

SCEE-BS-HS-TEM-0014 Scaffold Register

RTIO HSEQMS Standard C4 Working at Heights

RTIO-HSE-0090983 Working at Heights Work Practice

RTIO-HSE-0164172 Iron Ore - Scaffold Guidance Note

RTIO-HSE-0183535 Iron Ore - Scaffold Checklist (Mandatory For Every Scaffold)

## Barricading

Temporary hard barricading shall be in place around all areas where people may be exposed to hazards, such as:

- Being struck by materials (Falling Objects): Under EWP baskets and under personnel working at height;

- Falling into open excavations or penetrations;

- Cranes & EWP’s (Around slew radius & drop zones);

- Hot Work areas;

- Blind side of a truck whilst being loaded/unloaded; and

- Any area that requires the exclusion of unauthorised personnel.

The following types of barricading shall be used:

- Hard barricading shall be utilised when an area is required to be completely enclosed to prevent unauthorised entry into a hazardous area (Falling Objects, Crane Slew Radius (not required with Franna Cranes) etc.);

- Traffic cones shall be used around the slew area of an EWP main body or used for temporary delineation to highlight the presence of trip hazards of plant, equipment or materials;

- Hard barricading shall be used around all trenches, excavations, the open sides of all floors, floor openings, stairwells, roofs, platforms and walkways from which a person can fall any distance or where personnel are working above.

Barricading should be kept at least one metre from the edge of an open trench, excavation or danger area, and must provide a clear visual warning.

Personnel shall comply with the following:

- At no time are personnel allowed to cross over, under or through barricades;

- When using hard barricading (e.g. crowd control barricading), panels shall be locked together. Hard Barriers are to be rigid and continuous;

- Barricading and signage shall be inspected & maintained daily;

- Authorisation from the supervisor or delegate must be obtained prior to removing barricading that was erected by another work group;

- When the task the barricading has been erected for is complete or has changed, all signage shall be removed, and the barricades packed away;

- Barricading shall be removed when no longer in use.

Barricade Signage/Information Tags

- Additional signage shall be used to identify specific hazards e.g. Grid Mesh removal, Falling Objects, Hot Work, Excavation, Workers Above, or Crane Slew Zone etc.;

- Barricading should have signage at least every 6 metres identifying its use;

- Information tags shall be in place on all sides of barricading, detailing works in progress and a contact name and contact details of the supervisor (i.e. radio channel, phone number)

- Exclusion zones shall be barricaded without an entry point and appropriate signage attached (e.g. Exclusion Zone, Do Not Enter; Drop Zone, Do Not Enter);

Where personnel are required to access or egress a barricaded area, entry and exit points used shall be clearly identifiable with entry gates and with entry / exit signs. Positive communication is essential and required by all parties involved to make sure entry is safe.

The following personnel are responsible for the barricade:

- Crane slew zone - Crane Operator

- EWP drop zone - Spotter

- Blind side of truck - Truck driver

- General work area – Supervisor

Refer:	SCEE-BS-HS-PRO-0021 Trenching and Excavating

RTIO-HSE-0049734 Barricading, Demarcation and Signage Work Practice

## Inclement Weather

In any situation where inclement weather conditions do or are likely to affect safe work, affected workers and the SCEE supervisors shall consult and seek the best method for completing work safely or shall seek alternate safe work if available. Contingency plans should be reviewed as the project progresses to identify work areas or tasks that can be performed during inclement weather.

Lightning alerts are communicated via Control (Site radio) and/or via the website: http://mining.weatherzone.com.au.

In emergencies work may continue, or as agreed in circumstances such as when provided with systems or procedures (i.e.to unload trailers or complete concrete pours), during periods of inclement weather where it is agreed that to not move or protect equipment or goods, substantial loss may occur, or substantial risk is present.

Refer:	SCEE-BS-HS-PRO-0011 Inclement Weather Management

RTIO-HSE-0166185 Lightning Safety Work Practice

RTIO-HSE-0298258 RTIO PMO Safety Lightning Plan Document

## Traffic Management

Traffic Management Plans will be required for situations where activity will necessitate a full or partial road closure or traffic diversion or detour that will impact on operational activities and traffic movement.

Plans must normally be submitted to the client for approval a minimum of 72 hours prior to the activity commencing.  All drivers and operators must comply with Traffic Management Plans as per their scope of work.

SCEE shall work in compliance with Client Traffic Management procedures.

Refer:	SCEE-BS-HS-TEM-0046 Traffic Management Template

RTIO-HSE-0049645 Vehicles and Driving Work Practice

RTIO-HSE-0049734 Barricading, Demarcation and Signage Work Practice

## Use of Power and Hand Tools

SCEE supervisors shall ensure that all personnel are supervised in the selection, use and maintenance of power and hand tools they are required to use.  All personnel will be informed on the use and maintenance requirements of hand and power tools.

During the SCEE Site Induction and periodically at Pre-start and Toolbox Meetings personnel will be reminded of their responsibilities before and during the use of power and hand tools.

These responsibilities include but are not limited to:

Ensuring that power tools have a current quarterly test tag attached

Checking the tool prior to use for damage to the tool or power lead

  - Any equipment that is defective, damaged or untagged will not be used and such equipment will then be tagged with an Out of Service tag and reported to the Supervisor.

Ensuring that all mains powered electric tools are fitted with a “Dead man” switch

Wear the appropriate and correct PPE for the task

Ensure the correct tool is used for the task and is used as per the manufacturer’s recommendations

Ensuring that all 240 volt (or above) power tools are connected to an approved industrial type of RCD

Ensure the  correct guards are fitted and in good condition

Work pieces are clamped where possible

Ensuring that tools are not lifted or carried by their electrical leads

All power tools will be required to undergo quarterly testing and tagging procedures as per AS/NZ 3760 and will be recorded on the project electrical equipment register. The quarterly test tag colours are defined as:

Red 	-	December to February

Green	-	March to May

Blue	-	June to August

Yellow	-	September to November

Refer:	SCEE-BS-PO-TEM-0038 Electrical Equipment Register

RTIO-HSE-0049669 C2 Electrical Safety Work Practice

RTIO-HSE-0248195 Safe Electrical Work Practice

RTIO-HSE- 0022651 Hand and Power Tool Safety Work Practice

### Electric Arc Welding

Welding machines brought to site shall be inspected to meet the requirements of the Iron Ore (WA) Welding Machine and Accessories Checklist RTIO-HSE-0087155 prior to being authorised for entry onto site.  A copy of the completed Iron Ore (WA) Welding Machine and Accessories Checklist RTIO-HSE-0087155 shall be provided to area electrical supervisor.

All welding machines shall have an approved Voltage Reducing Device (VRD) fitted prior to use. The VRD shall have a current inspection tag displayed. All electric welders shall have a current test tag fitted on the primary cord. The welder should be marked what category welding environment it had been tested to every 3 months.

Refer:	RTIO-HSE-0087155 Welding Machine and Accessories Checklist

RTIO-HSE-0049671 Welding Work Practice

### Hand Held Drills

The following shall be applied when undertaking drilling activities with Hand Held Drills:

- Operators are to ensure all loose items are clear of rotating parts.

- Personnel drilling should ensure good body positioning and the material is secured in the event of the drill bit biting.

- Steel >8mm required to be drilled, consideration should be given to the use of a Rota Broach.

- Consideration should be given in the use of a drill mate.

### Hand Held Grinders

The following shall be applied when undertaking grinding or cutting with a Hand Held Grinder:

- Angle grinders shall have fail safe switches (Clutch) to prevent kick back;

- Operators are to ensure all loose items are clear of rotating discs.

- Where grinding or cutting activities are to be undertaken, the direct area should be clear of combustibles and screens used as required.

- Cutting wheels shall not be used for grinding work.

- The grinder shall not be put down with the disc still in motion. Discs shall not be stopped or slowed by applying pressure to its face or edge.

- Personnel shall ensure that the grinding and/or cutting disc RPM has matched or exceeded the RPM of the handheld grinder.

- The correct disc is used for the task.

- Discs should be inspected regularly for integrity.

- Personnel grinding or cutting should ensure good body positioning and the material is secured.

- The cutting and grinding of drums is strictly prohibited.

- Handheld grinders shall have handles attached. Handles may only be removed upon completion of a risk assessment (i.e. JHA). Two hands used to hold the grinder at all times.

- Nine-inch grinders are prohibited.

- Double eye protection (i.e. full face shield – medium impact rated, in conjunction with safety glasses or mono-goggles) shall be used when grinding or cutting.

- Discs less than 1.6mm thick shall not be used.

### Rota Broach/Magnetic Drills

The following shall be applied when utilising a rota broach or magnetic drill:

- Atra Ace WO 3250 Magnetic Based drills are prohibited from use on this project;

- Operators are to ensure all loose items are clear of rotating parts. Personnel drilling should ensure good body positioning and the material is secured.

- Information Tag to be placed on the power source end of the electrical extension lead highlighting that a Rota Broach is in use and not to disconnect the power supply.

- Operators are to ensure the unit is secured in the event of non-supply of power to the unit.

- Information tag to be placed at the power source stating, “Do not turn off, Rota Broach in use”.

- Operator to ensure the drill is off waits for broach to stop before lifting guard and clearing swarf from broach. Gloves to be worn and brushes or other tools used rather to clean swarf.

- Double eye protection (i.e. full face shield – medium impact rated, in conjunction with safety glasses or mono-goggles) shall be used when using a Rota Broach/Mag Drill.

- Where a rota broach is recommended and there is no structure to place the units’ base, consideration should be given to the use of a mobile base plate.

## Banned Tools Onsite

The following tools are considered to present an unacceptable level of risk and as such are not permitted to be used on site.

9 inch (230mm) grinders

1.5mm knifing/cutting discs

Open blade knives and trimming knives (i.e. Stanley etc.)

Hardened steel hammers

Crow bars without a cap at one end

Oxy / Acetylene systems not fitted with Flash back Arrestors;

Adjustable spanners or shifters

Over-centre type Binders

Atra Ace WO 3250 Mag Drills.

Manual torque multiplier or home-made tools

Refer:	RTIO-HSE-0132630 Banned and Restricted Tools

  - Plant and Equipment – Other

    - Installation/Commissioning/Demolition

Plant must not be installed, commissioned, dismantled or decommissioned unless it has been established that it is safe to do so and will be done without risk to the health and safety of any person at the workplace.

A worker who installs, assembles, constructs, commissions or decommissions or dismantles the plant must be deemed a competent person. They must be provided with all information necessary to eliminate or minimise risks to health, safety and environment.

The processes for the installation, construction, commissioning, decommissioning and dismantling of plant will include inspections that ensure that risks associated with these activities are monitored and reduced to ALARP.

All Plant and Equipment prior to the processes of installation, construction, commissioning, decommissioning will be barricaded off to the general workforce to ensure alterations or interference with the plant is not undertaken without Site Management acknowledgement.

    - Fit for Purpose

Plant must be used only for the purpose for which it was designed. Where plant is not used for its designed purpose a risk assessment must be completed by a competent person to determine if there is an increase to the health or safety of workers.

Plant that is not in use at the workplace must be left in a state that does not create a risk to the health or safety of any person.

Guards must be in place that are of solid construction and securely mounted to the plant. Guards should be properly maintained in order to not create a risk in itself. If guarding is removed from the plant, then controls must be in place to ensure it can't be restarted unless the guarding is replaced.

    - Plant and Equipment Controls

Operational controls must be easily identified on the plant to indicate their nature, function and direction of operation and located so they are readily and conveniently available to be operated by each person using the plant. The controls must be able to be locked in the off position to enable the disconnection of all motive power.

Operational controls must be guarded, controlled or located so that they cannot be unintentionally activated.  Operation controls must be able to be locked in the off position when required.

Any controls not identifiable or guarded must be reported to your supervisor and the area secured.

    - Maintenance/Inspection of Plant

- Maintenance, inspection and, if necessary, testing of the plant must be carried out by a competent person in accordance with the manufacturer's recommendations. If there are no manufacturer's recommendations then the maintenance, inspection and/or testing must be conducted in accordance with the competent person's recommendations or in relation to inspections as approved by Site Management.

## Fire Prevention

SCEE Project Management recognises that fire is the single most preventable occurrence that has the potential to cause the greatest loss.  SCEE Project Management is committed to a fire prevention program that shall consist of:

Fire extinguishers being located at SCEE work locations

Fire extinguishers being inspected and tagged on a six monthly basis by fire equipment service company or qualified personnel on site

Hot work permits system being adopted and implemented for all hot works by SCEE workers as per site requirements

Portable Fire Extinguishers will comply with AS 1841 and be maintained to AS 1851.1

Refer:	SCEE-BS-HS-WIN-0029 Fire Extinguisher Inspection

SCEE-BS-HS-PRO-0015 Hot Work (High Risk)

SCEE-BS-RM-WIN-0004 Gas Bottle Handling and Storage

## Electrical Safety

Project Management shall ensure:

All electrical work is carried out in accordance with the relevant state or territory electrical regulations and AS/NZS 3000 Wiring Rules

Only qualified electricians carry out or supervise electrical work

All portable electrical equipment and electrical leads are inspected and tagged on a quarterly basis

All portable electrical equipment and electrical leads are visually inspected prior to each use

Residual Current Devices (RCD’s) are used on all electrical circuits from the General Purpose Outlet (GPO)  including portable generators

All electrical leads and extensions are routed off the ground or along edges where possible, to remove the tripping potential and to prevent damage and ingress of moisture

Any person carrying out high voltage work completes the Client’s Permit Procedures

The strictest compliance to the Client’s isolation and tag-out procedures are adhered to

All Electrical Work shall be conducted in accordance with the Clients Electrical Specifications, Isolations and Permit to Work systems.

Refer:	SCEE-BS-HS-PRO-0005 Electrical Isolation and Tag Out

RTIO-HSE-0049638 Isolation Work Practice

RTIO-HSE-0049676 Electrical Appliances, RCD’s and Earth Continuity Relays

RTIO-HSE-0049677 Iron Ore (WA) High Voltage Installation Work Practice

RTIO-HSE-0248195 Safe Electrical Work Practice

### Electrical Isolations

When a circuit or equipment has been isolated by an Authorised person to enable work to be carried out on or adjacent to such equipment, an approved isolation lock is to be attached to the approved isolation device, danger tag attached to the lock signed and dated by the worker carrying out the isolation. If multiple persons are to work on the isolated equipment, an approved multi-lock device with individual locks and tags attached by each person shall be used.

- Danger tags and locks are to be securely fixed to the operating handle or button of the equipment so that there is no risk of their being accidentally dislodged or of an operator being able to operate the equipment without seeing the tag.

- No switch may be operated whilst a lockout device and danger tag is attached.

All electrical isolations shall be achieved by a physical air gap or by the use of a suitable locking device.

Prior to commencing work on any electrical equipment a test for dead shall be completed with a suitably calibrated device.

After any break in work or change of conditions a re-test for dead shall be completed prior to recommencing any electrical work.

A lock and danger tag may only be removed by the person whose signature appears on it, except that in the circumstances where the owner of the danger tag and lock is unavailable due to leave, illness or other cause, the Authorised Person or the delegate and appropriate client authority may remove the lock and tag, but only after complying with the following conditions:

•	They shall acquaint themselves with the reasons for the lock and tag being attached;

•	They shall carry out all necessary inspections and tests on the circuits/equipment to make certain that such may be operated without danger to person or equipment;

•	They shall ensure that all persons associated with the work are informed of their action;

•	Incident report shall be completed

No live work shall be undertaken. At all times equipment is to be isolated prior to conducting work in accordance with the site isolation procedures.  In addition to this:

•	All electrical equipment must be treated as live, until proven de-energised.

•	A “Test Before You Touch” in accordance with isolation regulations shall be performed to confirm there are no live exposed parts prior to commencing the work, and prior to recommencing the work where the work has been paused for any period of time.

•	All electrical isolations shall be locked out and performed by an authorised isolation officer.

•	Certified voltage testers shall be tested for correct operation before use and after use to confirm it is operating effectively.

•	Proximity Testers shall not be used to confirm the presence of live voltage;

•	Panel Volt Meters shall not be relied on to confirm an electrical part is de-energised.

Refer:	SCEE-BS-HS-PRO-0005 Electrical Isolation and Tag Out

RTIO-HSE-0049638 Isolation Work Practice

RTIO-HSE-0049676 Electrical Appliances, RCD’s and Earth Continuity Relays

RTIO-HSE-0049677 Iron Ore (WA) High Voltage Installation Work Practice

RTIO-HSE-0248195 Safe Electrical Work Practice

### High Voltage Work

High voltage works, isolation and switching must be performed as per RTIO’s High Voltage Installations Work Practice. High voltage equipment must be installed and operated, and accessed, by personnel authorised by the EPCM’s Construction Manager.

High Voltage is defined as voltage exceeding 1000 volts AV or 1500 volts DC.

People working within 10 metres of a powerline must be authorised by a Vicinity Permit. Warning signs and barricading must be erected to prevent unauthorised entry. Safe working distances for people and equipment must be assessed against the requirements of the High Voltage Installations Work Practice.

Minimum safe approach and working distances to exposed high voltage equipment shall be maintained as follows;

| Voltage | Ordinary Person (mm) | Ordinary Person with a Safety Observer (mm) | Instructed or Authorised Person (mm) | Mobile Plant Operated By: Instructed or Authorised Person (mm) |
| --- | --- | --- | --- | --- |
| Up to and including 33kV | 6400 | 3000 | 700 | 1200 |
| 132kV | 8000 (v) 10000 (h) | 8000 | 1200 | 1800 |
| 220kV | 8000 (v) 10000 (v) | 8000 | 1800 | 2400 |

HV work may include the removal and de-termination of existing cable and re-installation and termination of new HV cable. Isolations shall occur at the relevant switch room and switch gear prior to any HV work.

Permits shall be used whenever access or isolations to switch room equipment is required.

For all High Voltage work:

- The permit recipient will be appropriately trained;

- A permit shall be raised;

- The circuit shall be isolated to confirm the circuit is de-energised and earthing interlocks, where fitted, are engaged.

Refer:	RTIO-HSE-0049677 High Voltage Installations Work Practice

RTIO-HSE-0240073 High Voltage Isolation Safety Work Practice

### Electric Shocks and Events

All electrical shocks and events require immediate attendance at the medical centre.

Electric shock events must be reported to the SCEE Site Manager, Project Manager, HSE Advisor, the Nominated Client Electrical Supervisor and the Client HSE Manager immediately regardless of the severity. The nominated SCEE electrical representative for all suspected electric shock events is the SCEE Training Manager. The Training Manager is to be notified following all actual or suspected electric shock events and will participate in the incident investigation. All notifiable incidents will be reported to the applicable regulator through the Client HSE Manager or SCEE Electrical Representative within the required time frames.

If you come across a person receiving an electric shock:

Assess the situation – never put yourself at risk

Take precautions to protect yourself and anyone else in the vicinity

Have someone report the incident to the supervisor and contact emergency services

If possible, disconnect the supply

Assess the injuries and if safe to do so, move the casualty to a safe area

Administer first aid if trained

Ensure the incident scene is not disturbed

Any person who has received an actual or potential shock is to attend the medic or hospital for an ECG to monitor heartbeat rhythm.

Refer:	SCEE-BS-HS-PRO-0004 Event Management and Investigation

SCEE-BS-HS-WIN-0006 Event Reporting

SCEE-BS-HS-GUI-0001 Event Notification Guide

RTIO-HSE-0063098 Non-Conformance, Incident and Action Management

### Electrical Fires

Electrical fires may result from:

The overheating of circuits

Machines running too slowly for long periods due to insufficient electrical power

Machines driven beyond their intended capacity

Oil and dust being allowed to accumulate on motors etc.

Overheating of equipment by:

  - Too much equipment on the same circuit

  - Too large a fuse being used in a circuit

Only use a CO2 or dry chemical extinguisher on an electrical fire. Fire extinguishers must be kept near main switchboards and substations as a minimum.

Generally CO2 is not used on RTIO sites due to the asphyxiation risks.

### Controlling Electrical Hazards

SCEE management shall regularly discuss the controlling of electrical hazards in the workplace commencing with the SCEE Site Induction and continuing throughout the project in Prestart and Toolbox Meetings. Workers will be advised of the following controls that can be used in the workplace:

Visually inspect all electrical equipment to ensure it is correctly tagged for the quarter

All portable electrical equipment shall be used in conjunction with a portable RCD unit

Do not use electrical devices near water

Unplug equipment correctly (do not pull cord!)

If a circuit breaker has tripped, only reset it once, after which the supervisor should be advised

Use ‘out-of-service’ tags and report faulty equipment

Use the right equipment and fuses

Adhere to isolation requirements and work procedures

Read and heed warnings

Increase your resistance to shock by wearing the issued safety boots. Higher resistance leads to lower current.

- Refer:	RTIO-HSE-0248195 Safe Electrical Work Practice

## Substations

Only authorised persons will enter and work inside a substation. Substations must always be left locked when unattended and clear safety glasses must be worn when inside substations with a minimum of CAT 0 clothing being worn

No persons are to work alone inside an energized switch room and shall have the RTIO Sub Station Entry Induction.

All electrical work required to be recorded must be entered into the Electrical Record Book and the appropriate Client Electrical Supervisor informed.

## Noise Control and Occupational Hygiene Monitoring

On the project the use of IPod’s, MP3’s and similar devises fitted with ear phones/pieces are not permitted.  The use of mobile telephones is restricted to management and supervision in areas other than crib areas, unless approved by the Project Manager.

The continuing assessment of worker noise exposure levels will be an ongoing process throughout the project.  Project management shall ensure that all plant or equipment identified as generating excessive noise above prescribed levels will be properly identified and signposted. Exposure times and or time weighted averages shall be discussed at prestart and toolbox meetings along with the appropriate protective hearing devices and instructions on their use.

Worker awareness and monitoring of occupational stressors including vibration, dust, fibrous materials, chemical exposure, ergonomics and manual handling will be undertaken by SCEE Project Management.

Refer:	SCEE-BS-HS-PRO-0009 Noise Control and Hearing Protection Procedure

## Health Surveillance

Health surveillance may be required in the event personnel are considered to be at risk as a result of exposure in the workplace to a hazardous substance as outlined in applicable legislation. All personnel have the opportunity to undergo a health risk assessment every twelve (12) months if requested.  The risk of exposure to harmful substances shall be identified and assessed using appropriate control measures in the project risk assessment.

If the outcome of the project risk assessment determines surveillance is required health surveillance records shall be maintained confidentially in worker’s personnel files, with ongoing surveillance coordinated by the Human Resources department.

## Fibrous Materials

Whilst fibrous materials occur naturally within geological formations in certain regions, no naturally occurring fibrous materials are likely to be encountered during the project works.

Mineral fibres often occur in large visible bundles, but individual fibres and fibrils are not visible to the naked eye. Mineral fibres under the category of ‘asbestos’ fall into the classification of fibrous minerals. Asbestos is a hazardous material that poses a risk to health by inhalation if the asbestos fibres become airborne and personnel are exposed to airborne fibres. The definition of an asbestos fibre is a fibre with a diameter (width) less than 3 microns, a length greater than 5 microns and a length to width ratio greater than 3:1.

Exposure to asbestos fibres is known to cause mesothelioma, asbestosis and lung cancer. It should be stressed however, that risk of health impact from fibre inhalation is dose related, i.e. the greater the dose the greater the risk of disease (other factors such as smoking will increase the risk). The onset of disease usually occurs many years after exposure (chronic health effects).

It should also be stressed that not all fibres, because of their size and dimension have the ability to enter the lungs. Large fibres are trapped within the upper respiratory tract where they are removed by natural body processes.

The Safe Work Australia Code of Practice - How to Safely Remove Asbestos (2011) should be referred to whenever removal is identified as the best control measure.

SCEE Project Management shall ensure that all workers are made aware of the risks associated with fibrous materials, the risk control measures and their role within that procedure, during the SCEE Induction.

Refer:	RTIO-PDE-0062061 Fibrous Materials Management Plan for RTIO

RTIO-HSE-0138640 Asbestos Management Work Practice

### Hazardous Paint Removal

SCEE is aware that the paints on old structures may contain hazardous substances such as lead, arsenic, cadmium, chromium and asbestos. Any Works requiring the original surface treatment to be disturbed (removal of flaking paint and coatings, or other similar activities which may result in the creation of dust and/or debris) should be considered hazardous and managed accordingly.

Depending on the extent of Works, the SCEE must take five (5) to ten (10) random paint samples from each asset and test for hazardous substances. The testing must be conducted by a NATA certified test facility. Testing must be completed within eight (8) weeks from the Commencement Date to establish which assets will require hazardous paint removal.

If identified hazardous paint removal must be performed by suitably qualified workers (using appropriate PPE and exercising the skill expected of a professional contractors) and otherwise in accordance with;

  - [AS4361] Guide to Hazardous Paint Management Lead and other hazardous metallic pigments in industrial applications;

  - [NOHSC: 2015(1994)] National Code of Practice for the Control and Safe Use of Inorganic Lead at Work;

  - [P-0000000-0000-H-GUI-00001] H01 RTIO Lead Management Guidance Note;

  - the Contractor’s approved Hazardous Paint Management Plan; and

  - applicable Laws.

All waste and debris generated from SCEE’s works and activities involving hazardous paint must be prevented from leaving the relevant works area by suitable physical barriers or mechanical means.

SCEE shall not remove the encapsulation until it has collected, removed and disposed of all dust and/or debris in accordance with AS4361 and the manufacturer’s recommendations (where applicable) and otherwise to RTIO’s representative’s satisfaction.

SCEE will ensure that all surfaces are wiped down, inspected and cleared and left in a suitable condition, before applying any new protective coating, paint or both. SCEE will prepare and maintain waste tracking documentation for all hazardous materials (including hazardous paint waste) created and disposed of during the project.

Upon SCEE providing the RTIO five (5) business days’ notice that the relevant works or activities are complete, RTIO will engage an independent certified occupational hygienist to complete clearance inspections on all surfaces where hazardous materials (including hazardous paint) have been removed by SCEE, including a detailed visual inspection and verification (through swab or similar methods) of all the relevant surfaces.

## Working Alone

A person is “working alone” when they are physically on their own, when they cannot be seen or heard by another person or when they cannot expect a visit from another worker or member of the public for some time.

The following examples indicate situations where workers may be exposed to working alone:

A person who is working alone in an office

A person working after others have gone home

A supervisor assessing a future job site/scope by themselves

An worker working by themselves in a remote area of a conveyor system

Travelling alone in company vehicles for extended periods of time

SCEE workers who are directly supervised by third parties (e.g. Client personnel) may still be working alone if the person supervising them leaves the direct area of work for long periods of time while the work is being performed.

Requirements to develop systems of work for personnel working alone are outlined in the SCEE Work Instruction on Working Alone.

Refer:	SCEE-BS-HS-PRO-0024 Working Alone

## Simultaneous Work Activities

At times SCEE personnel will be working in areas with simultaneous work activities occurring. For example shut down operations with other contractors or carrying out installation activities where normal construction works are underway. In order to assist in the management of simultaneous work activities in such a way as to eliminate or minimise the risk of fatalities, injuries or events, the following should be discussed and implemented where practicable:

Coordination meetings with relevant contractors to discuss work schedules

The Client may be advised of proposed work activities at weekly progress meetings

Field supervision (SCEE and other contractor) may have interface meetings on the work site to delineate work areas and coordinate movement

Traffic Management Plans, crane activity, safety notices and bulletins regarding planned activities are communicated during prestart and toolbox meetings

Permit and area access restrictions may be designated

Barricades, flagging and bunting can be set up and information tags used to communicate work and access details

Where workers are working above or below other workers, supervisors shall encourage the communication between work groups and ensure that the separate work areas are appropriately set up. This may include providing overhead protection, erecting a barricaded drop zone, using drop sheets and removing personnel from potential line-of-fire positions.

SCEE shall undertake a Critical Risk Management - Critical Control Checklist for any falling object risk prior to commencing work.

The schedule for interface meeting should be agreed between all stakeholders prior to the work commencing. Interface meetings would normally be held fortnightly or monthly but in environments such as shutdowns, work adjacent to operating plant or equipment or in dynamically changing environments; meetings will be more frequent.

## Management of Change (MOC)

SCEE recognises uncontrolled change at the work front as a significant risk.

Measures to reduce the risk of incidents, injury, equipment failures and damage arising from change events must ensure:

Change is managed in a structured and uniform manner in line with the SCEE and client HSE management systems.

Work crews are included and have input into the change management process

A Risk Assessment is conducted

Controls deliver the required outcome and risk is reduced as low as practicable

No new hazards to persons, equipment or the project are introduced

Documents are reviewed and updated in line with the appropriate document control procedures

- Any permanent or temporary changes to the SCEE work front shall only proceed if:

A Risk Assessment is completed to assess the potential impacts of the change. For minor change, this could be a Take5. If the residual risk rating is high or above work must not proceed until a formal risk assessment has been completed.

A plan is developed by supervision and the work crew that clearly specifies the actions and timescale for the change and the control measures to be implemented. This could be a formal risk assessment or an amendment to the Safe Work Method Statement or JHA.

Refer:	SCEE-BS-HS-WIN-0009 Workfront Management of Change

RTIO Standard – Element 11 Management of Change Process

## Hazardous Substances

SCEE shall comply with the relevant Australian Standards and Codes of Practice, SCEE and RTIO procedures to ensure the following:

- Transport shall be in accordance with the applicable Transport of Dangerous Goods legislation and shall be performed by licensed contractors

- The storage of hazardous goods shall be in accordance with the relevant legislation and as per the information on the SDS

- First aid requirements shall be supplied as per the information on the SDS

All proposed hazardous substances required for the project shall be submitted to the RTIO CMS Advisor via the RTIO Chemical Control Checklist RTIOHSE-0036574.  Substances can only be brought onto site once approval has been granted.  Approval shall be given via the use of the RTIO Chemical Risk Assessment and Request Flow Chart.

A hazardous materials register will be maintained by site. Safety Data Sheets (SDS) for hazardous materials stored in the place of work will be maintained on site.

Refer: 	SCEE-BS-HS-WIN-0021 Hazardous Materials Handling and Storage (High Risk)

RTIO-HSE-0121452 Hazardous Substances Work Practice

RTIO-HSE-0036574 Chemical Control Checklist

## Risk Assessment and Control of Hazardous Substances

In the event that certain hazardous substances are required, the Project Manager will ensure that a risk assessment is completed prior to the introduction of any such substance or product onto site. Factors such as the SDS, manufacturer’s information, nature of work, experience of workers and the potential health effects of each substance are to be considered in the completion of the risk assessment.

Refer: 	SCEE-BS-HS-WIN-0021 Hazardous Materials Handling and Storage (High Risk)

## Storage and Disposal of Hazardous Materials

All hazardous materials or chemicals shall be stored in accordance with the manufacturer’s recommendations and statutory requirements in appropriate, correctly labelled containers and shall be accompanied by a SDS which will be made readily available to workers.

Containers of fuel are not to be stored near a flame or any other source of ignition and there will be no smoking in the vicinity. Containers shall be regularly inspected by a competent person.

All equipment excluding mobile plant which holds more than ten litres of fuel and has the potential to leak will be placed on drip trays. Equipment which holds less than ten litres of fuel will be stored suitably to reduce the risk of leaks and spills.

Emergency showers and eye-wash stations shall be available by RTIO and be located in appropriate positions within close vicinity of the hazardous substance use and / or storage. RTIO shall ensure that locations for emergency showers and / or eye-wash stations are appropriately selected, equipment signposted and regularly maintained in accordance with: AS/NZS 4775: Emergency eyewash and shower equipment.

Training in the use of emergency showers and / or eye-wash stations shall be provided through area familiarisation (i.e. walk-through’s) and / or site inductions.

Disposal of hazardous materials shall be in accordance with legislative requirements and manufacturer guidelines. Only contractors with the appropriate license shall transport and dispose of such material.

Refer: 	SCEE-BS-HS-WIN-0021 Hazardous Materials Handling and Storage (High Risk)

## Hydrocarbon and Other Chemical Management

A site register of all hazardous substances with appropriate SDS is maintained and stored as per SDS recommendation and as required under AS 1940:2004. The SDS register will be located in the site office, stores and hazardous goods container. Updates to the register are advised to workers at Toolbox and/or Pre-start Meetings.

The Construction Manager shall ensure that all bunds, storage and fire precautions shall be in compliance with the applicable Dangerous Goods (Handling & Storage) Regulations and AS 1940:2004 Storage and Handling of Flammable and Combustible Liquids.

Containers of fuel will not be stored near a flame or any other source of ignition and there will be no smoking in the vicinity. Containers shall be approved for carrying flammable liquids. Bulk fuel storage areas will be poly lined.

Hydrocarbons and chemicals shall have 110% secondary containment facilities. Authorisation shall be obtained from the client prior to commencing any job that requires the handling of hydrocarbons or chemical in the vicinity of water bodies and a Job Hazard Analysis (JHA) shall be conducted.

Suitable firefighting equipment shall be available and regularly inspected by a competent person.

Incompatible substances shall not be stored together.

## Hydrocarbon and Chemical Spill Management

Hydrocarbon and/or chemical spills shall be managed on site according to the following flowchart.

It is the responsibility of the SCEE Project Manager to ensure that in the event of a spill / release there is immediate access to appropriate clean up equipment and absorbent materials. For example:

- Inert absorbent material such as vermiculite, clay, sand or kitty litter in sufficient quantity to absorb the spill (or these goods would be stored in a bunded area large enough to contain the spill)

- Shovels will be taken to the spill area but will be kept on hand at areas such as fuel storage depot all times

- Barricading/Bunting/Witches Hats

The removal and disposal of contaminated soil shall be arranged in accordance with contractual requirements, disposed of in compliance with site requirements and applicable regulations.

SCEE will ensure that portable generators, welders or stationary engines comply with site standards.

SCEE refuelling trailers used for transportation of hydrocarbons shall be fitted with appropriate spill kits and drip trays. Additional spill kits shall be located at the SCEE stores and made readily available for use wherever hydrocarbons and chemicals are stored and/or used.

## Pollution /Emissions

Due to the nature of work performed, SCEE is considered a minor contributor of emissions. Potential sources of emissions have been identified as primarily light vehicles, site based load shifting plant and small internal combustion engines. To ensure that emissions are maintained at satisfactory levels, all equipment is regularly maintained in accordance with manufacturers’ recommendations.

The burning of rubber and plastic products, waste oils or any waste material is not permitted. Any inadvertent emission of pollutants within or from the work site will be reported to the HSE Advisor. Areas which are suspect or known to be contaminated sites will be recorded.

## Dust Management

In accordance with SCEE and client procedures, SCEE will implement reasonable measures to:

- Ensure that a proactive approach to dust minimisation is taken for all work activities

- Avoid nuisance impacts of dust on the workforce and surrounding land users.

The source of any nuisance dust will be identified and where practicable, dust control practices will be modified to reduce or eliminate ongoing risk. The potential for dust generation shall be reassessed periodically.

Dust control measures include:

- Road watering of unsealed access roads and road verges of sealed roads

- Watering of unsealed lay-down areas

- Speed restrictions of roads

- Minimising dust generating activities during dry and windy conditions

Refer: 	SCEE-BS-HS-TEM-0002 Dust and Fibrous Materials Management Plan

## Protection of Flora and Fauna

SCEE personnel shall comply with the SCEE processes and Work Instructions. This includes the following:

- Necessary approvals will be obtained prior to the removal of native vegetation

- SCEE shall not otherwise remove or damage any trees, plants or animals during the course of work activities and shall minimise the need to disturb any area

- Significant tree communities or individual trees to be retained on site works, will be clearly identified through appropriate tagging

- All work activities will be confined to gazetted sites. Under no circumstances shall any personnel or vehicle travel on other than existing formed roads

- Fauna deaths and feral animal sightings shall be reported to the client

- No pets or other animals are permitted on the project site

- Firearms are prohibited on site

The site environmental representative will be contacted in the event that:

- Fauna relocation is required

- Road kill of cattle or fauna occurs (fauna kills will be reported as an environmental event)

- Feral animals are sighted within the Project area

SCEE will assist the Client in fauna preservation and protection throughout the Project.

## Soil and Water Conservation

SCEE shall take all precautions to prevent the discharge of any water containing contaminants into adjacent waterways.

Such precautions shall include:

- Maintenance of plant and equipment to be performed in an approved area

- Oil separation traps shall be used before discharge of drainage from maintenance activities

- Liquid waste will be collected in holding tanks and removed from site for recycling

- All oil or fuel spillage shall be cleaned up immediately

Refer:	RTIO-HSE-0076061 Notification of Environmental Incidents Procedure

RTIO Standard Element 14 – Non Conformance, Incident and Action Management

## Waste Water Management

SCEE will utilise client supplied amenities or if these facilities are unavailable disposed of waste water utilising a licensed waste disposal contractor.

## Waste Reduction and Disposal

Non Hazardous Waste

The company places significant emphasis on waste management, waste minimisation and the concept of cleaner production. The SCEE Project Manager is responsible for ensuring that environmental management procedures are followed during the project. The SCEE HSE Advisor will perform the monitoring activities required for effective Waste Management. These activities will include but not be limited to:

- Ensuring waste storage areas are correctly designated and identified

- Ensuring waste items are segregated appropriately

- Ensuring skips and other bins are not overflowing and are covered when necessary

- Ensuring waste storage containers and facilities are correctly labelled

- Promoting waste storage and disposal procedures through Toolbox and Pre-start meetings

- Ensuring all waste is disposed of in accordance with the relevant site procedures, Regulations and Codes of Practice

SCEE will consult with the site environmental representatives to confirm any site specific segregation and recycling requirements along with alternative waste disposal methods.

Where facilities are made available, the following work materials will be categorised:

- Organic (e.g. food scraps, low grade paper)

- Combustibles (e.g. wood, cardboard, paper)

- Recyclables (e.g. glass, paper, aluminium, plastic bottles, milk cartons)

- Scrap steel

- Batteries (not stored on site)

- Chemical waste

- Oil Waste – including contaminated rags

Refer: 	SCEE-BS-HS-TEM-0007 Waste Management Plan

RTIO E15 Hazardous Materials & Non-Mineral Waste Control and Minimisation

Hazardous Waste

Where hazardous waste is generated as part of SCEE activities, it shall be disposed of in client provided facilities, or if these facilities are unavailable disposed of using licensed hazardous waste disposal contractors.  Relevant records of types and quantities of waste removed by this method shall be maintained and reported in line with RTIO environmental management procedures and requirements.

Refer: 	SCEE-BS-HS-TEM-0007 Waste Management Plan

RTIO E15 Hazardous Materials & Non-Mineral Waste Control and Minimisation

## Weed Management

SCEE shall ensure all ground engaging, earthmoving or tracked equipment is cleaned before arrival on site to remove all dirt, stones or vegetation material to assist in the prevention of weeds and plant transfer from other regions.

All other SCEE equipment, including light vehicles, will be in a clean state and free from excess dirt or dirt material when presented on site. Weed hygiene certificates or the equivalent will be presented where required as a pre-mobilisation deliverable.  These documents will then be kept on file on site as auditable records.

SCEE understands that the wash-down of vehicles and equipment is prohibited on site unless a RTIO approved facility is available.

Equipment Hygiene Inspection Certificate RTIO-HSE-0085118 shall be completed and approved for all earth moving vehicles, heavy machinery and drill rig equipment entering and leaving Rio Tinto Iron Ore (WA) sites / projects (one certificate per vehicle / equipment).

Refer:	RTIO E14 Land Disturbance Control and Rehabilitation

RTIO-HSE-0085118 Equipment Hygiene Inspection Certificate

## Surface Water/ Ground Water Management

The SCEE scope of work does not involve work activities that cause surface water run-off or requires the installation of culverts, drains or bridges or in ways that impact natural drainage systems. Should the SCEE scope of work change to include such activities, reference will be made to project procedures and the appropriate written approvals shall be obtained.

SCEE will ensure that no SCEE generated construction materials are left in creeks, river beds or on the banks of watercourses. If potential contaminants are believed to have reached natural drainage channels SCEE will contact the RTIO environmental representative to initiate water-sampling procedures.

The servicing and maintenance of vehicles and equipment will be conducted, where possible, within a client approved facility.

Refer:	RTIO E14 Land Disturbance Control and Rehabilitation

## Indigenous Culture and Local Community Partnership

SCEE workers shall not disturb or remove any material of archaeological, anthropological, cultural or historic significance from any work site and this is communicated through the SCEE site induction.

All Aboriginal Heritage Sites shall be respected and not disturbed. SCEE shall immediately advise the RTIO environmental representative of any previously unidentified heritage sites or suspected Aboriginal heritage sites located during site works.  SCEE shall work with RTIO to ensure that these locations are protected from damage.

Refer: 	SCEE-BS-HS-TEM-0004 Cultural Heritage Management Plan

RTIO-HSE-0123835 Land Disturbance Work Practice

## Rehabilitation

SCEE will ensure that all temporary infrastructure and materials associated with the scope of work are removed from site.

The client environmental representative shall determine areas requiring rehabilitation prior to demobilisation and review the scope of rehabilitation works. The site environmental representative will also assess the quality of rehabilitation as part of the demobilisation process.

Refer: 	RTIO-HSE-0123835 Land Disturbance Work Practice

# Emergency Preparedness and Response

SCEE Project Management will follow the Client’s Site Emergency Procedures and work in conjunction with the Client Emergency Preparedness and Response plans. Emergency procedures covering medical emergencies, fire, natural disaster, bomb threat and cyclone situations shall be documented, practiced and included in worker training to effectively communicate and respond to any workplace event or emergency.

The aims of the emergency procedures are to:

Decrease the level of risk to life, property and the environment

Control any incident and minimise its effects

Provide the basis for training of all people who could be involved in any emergency

SCEE Site Management will coordinate with the Client an emergency response team (ERT) of volunteers that will be trained and participate as active members in the site Emergency Response Team. The SCEE Project Emergency Response Plan will be developed to integrate with the clients Emergency Response Plans.

Refer:	SCEE-BS-HS-TEM-0003 Emergency Response Plan

## Cyclone Preparedness and Response

The cyclone season for the project location will be determined to ensure preventative measures can be put in place. SCEE Project management are responsible for the clean-up, securing and tie down of plant, structures and materials within the SCEE scope of work.

The Australian Warning System is the approach to information and warnings during emergencies like bushfire, flood, storm, extreme heat and severe weather.

The System uses a nationally consistent set of icons, as per below.

Each warning level has a set of action statements to give clear and concise information about what to do. Below are some action statements you might see with a warning.

Advice (Yellow):

An incident has started. There is no immediate danger. Stay up to date in case the situation changes.

Watch and Act (Orange):

There is a heightened level of threat. Conditions are changing and you need to start taking action now to protect you and your family.

Emergency Warning (Red):

An Emergency Warning is the highest level of warning. You may be in danger and need to take action immediately. Any delay now puts your life at risk.

The three warning levels which move up and down to reflect the risk to your life or property before, during and after a natural disaster.

In addition, the Bureau of Meteorology issue cyclone information via radio using distinctive warning audio followed by three levels of alert

Tropical Cyclone Information Bulletin issued when a cyclone has formed but is not expected to impact within 48 hours

Tropical Cyclone Watch when a cyclone is expected to impact the coast within 48 hours but is not expected within 24 hours

Tropical Cyclone Warning when a cyclone is expected to impact the coast within 24 hours

SCEE is required to be 24-hours from full preparedness during cyclone season; nominally 1 November to 30 April.

In the event of a pending cyclone or demobilisation within the cyclone season SCEE will prepare their work area to make it cyclone safe and arrange with the RTIO Project Representative a walkthrough and signoff of the RTIO Site specific cyclone checklist prior to leaving Site.

Refer:	SCEE-BS-HS-TEM-0001 Cyclone Management Plan

RTIO Cyclone Procedure

# Incident Management and Response

## First Aid and Medical Treatment

SCEE Project Management is committed to providing first aid facilities and qualified first aid personnel that are easily accessible to all workers and able to manage minor injuries. Project Management shall ensure that:

There is a well-equipped first aid box in the site office

The first aid facilities comply with Australian Standards

Vehicle first aid kits comply with site requirements

The first aid facilities are checked regularly, and stock replenished as required

All first aid treatment is to be recorded in the incident register

First aid training is required to be undertaken by 5% of the site crew

All electricians will be required to hold a current CPR certification and receive CPR training every 12 months

All calls for medical assistance are to be made to the site paramedic

SCEE personnel will be accompanied to the site medic by HSE Advisor or Supervisor

Refer:	SCEE-BS-HS-PRO-0004 Event Management and Investigation

## Critical Incident Management

In the event that SCEE personnel are involved in a Critical Incident, Project Management shall refer to the Clients Critical Incident Management Plan for the Project. SCEE personnel shall be provided the following in the event of a critical incident:

Appropriate counselling services for as long as required

Access to the Worker assistance program

Notification or contact with their family or next of kin

Direct access to communications such as phone lines

Refer:	SCEE-BS-HS-PRO-0004 Event Management and Investigation

RTIO Element 14 - Non Conformance, Incident and Action Management

## Injury Management and Rehabilitation

SCEE will provide occupational rehabilitation for every worker who sustains an injury or illness at work.  Injury management will involve the coordinated use of medical, psychological, social, education and vocational measures to restore function or achieve the highest possible level of function for persons at work following an injury or illness.

All injury management and rehabilitation activities will be managed and coordinated by the Human Resources Department in accordance with the SCEE Workplace Rehabilitation policy and procedures.

The objectives are:

To assist the early, safe return to meaningful productive work following illness or injury

To establish rehabilitation as the normal course of action that begins when treatment is  started

To ensure accurate medical assessment, involvement of rehabilitation specialists when needed and to support the role of the treating medical practitioner

Site to provide a structured rehabilitation program. Instances where an worker requires assistance beyond first aid, the Human Resources department with the company’s rehabilitation provider and/or medical professional (s), shall prepare the rehabilitation program

To provide appropriate duties where necessary to assist the worker’s recovery whilst facilitating an early return to work

Refer:	SCEE-BS-HS-POL-0006 Workplace Rehabilitation Policy

SCEE-HR-IM-PRO-0001 Workers Compensation and Injury Management

SCEE-HR-IM-WIN-0002 Workers Compensation

RTIO Element 14 - Non Conformance, Incident and Action Management

## Incident Reporting

All workers (including subcontractors) working on the project shall report all incidents to their supervisor immediately.

The SCEE Event Notification Guide shall be displayed in the SCEE Site Office. SCEE Management shall be notified of all events in compliance with the Event Notification Guide.

Events must be reported formally for the following purposes:

To enable appropriate procedures to be implemented in order to prevent a recurrence

To enable a timely and appropriate investigation of circumstances of the event

To enable accurate and timely notification to insurers of any event that may be the subject of a claim

To comply with applicable legislation

For all events, an incident report shall be completed in STEMs by the end of the shift in which the event occurred.  The Incident Notification Guide outlines who must be notified if an incident occurs and the required notification time frames.

SCEE shall conduct a “for cause test” for alcohol and/or drugs following an incident which has occurred either in the workplace or at the accommodation villages. SCEE will endeavour at times to undertake this testing however may authorise an external party to assist as required.

Where testing is not achievable by either SCEE or an external party, site is to contact the HR Manager for further assistance.

All event and incident reporting shall also comply with Client Incident Reporting, Investigation, Corrective and Preventative Action and associated timelines.

Refer:	SCEE-BS-HS-PRO-0004 Event Management and Investigation

SCEE-BS-HS-WIN-0006 Event Reporting

SCEE-BS-HS-GUI-0001 Event Notification Guide

RTIO-HSE-00075010 External Notification of Health and Safety Incidents Work

Practice

RTIO Element 14 - Non Conformance, Incident and Action Management

## Incident Investigation

All incidents shall be investigated by SCEE Site Manager /Supervisor and HSE Advisor on site as a minimum requirement and include relevant personnel in the investigation as required.

Investigations shall be completed, and all associated evidence uploaded to the STEMS incident record within 7 days of the event.

An Essential Factors investigation will be conducted for all significant events. The Essential Factors investigation will be facilitated by RTIO or the SCEE HSEQ Manager (or delegate). As a minimum, an Essential Factor investigation will be conducted for the following events:

  - MTI

  - RWC

  - LTI

  - High Potential Near Miss

  - Procedural Breaches

  - Significant Environmental Event

An Essential Factors investigation may be conducted for other events at the request of the SCEE Executive Safety Committee or the Client. Investigation reports will be provided to the client as required or on request.

All identified corrective and preventative actions are to be assigned to the appropriate personnel and recorded against the STEMS incident record.

Refer:	SCEE-BS-HS-PRO-0004 Event Management and Investigation

SCEE-BS-HS-WIN-0007 Event Investigation

RTIO-HSE-00071439 Incident Investigation Work Practice

RTIO Element 14 - Non Conformance, Incident and Action Management

# Monitoring, Auditing and Review

SCEE conducts monitoring, auditing and review of HSE activities across its operations to ensure compliance with all Legislative, Company, Project and Client requirements and to identify any performance trends.

## Daily Safety Report

A Daily Safety Report shall be prepared each day and emailed to SCEE Corporate HSE and Project Management personnel for review not later than 9am the following day.

The report should as a minimum include activities performed by the HSE Advisors and list any events, incidents, provide details of hazards and controls, inspections carried out, permits organised, meetings attended, corrective actions underway, training conducted, BAC testing and results along with other information as deemed relevant on the day.

The Daily Safety Report shall be used for internal reporting only and is not for submission to the Client.

Refer: 	SCEE-BS-HS-TEM-0021 Daily Safety Report

## Monthly HSE Performance Report

The project HSE Advisor or Supervisor shall prepare a monthly HSE performance report each month and submit to Project Management and SCEE Corporate HSE. Reports shall be submitted by the 15th of each month.  The monthly performance report provides a summary of the project’s performance in the previous month and allows lead and lag performance data to be monitored and analysed.  The monthly performance reports are used to monitor individual site, divisional and company performance.

The monthly HSE performance report consists of:

- HSE KPI’s

- Incidents & Near Misses for the period

- Inspections & Audits for the period

- Management Site Visits

- First Swing Initiative Progress

- HazOb’s for the period

## Workplace Inspections

SCEE Project Management shall:

- Ensure supervisors and nominated workers are actively involved in conducting daily workplace inspections of all SCEE working areas and in accordance with site and this plans requirements

- Inspections to be recorded utilising Datatrack

- Utilise SCEE’s Workplace Safety Inspection / Checklist / Audit tools

- Ensure a copy of all workplace inspections are posted on all crib room notice boards and made available to the Client as required

Ensure Supervisors complete Daily Workplace Inspections

- Keep all completed checklists for the duration of the project

- Facilitate and encourage joint workplace inspections involving Client personnel and/or Elected Health and Safety Representatives

- Make regular observations supporting safe behaviours and discouraging at risk behaviours

- Record hazards identified during inspections in Datatrack

KPI’s shall be monitored and reports provided to site by the Corporate HSEQ Department

- Ensure Subcontractors participate in workplace inspections alongside SCEE Supervisors and nominated workers in line with KPI requirements.

- Daily Inspections conducted and recorded to ensure work is being undertaken in accordance with JHAs and SWMS.

Inspection Requirements

| Position | Type of Inspection | Frequency |
| --- | --- | --- |
| Project Manager | Weekly Inspection | 1 per week |
| Superintendent | Weekly Inspection | 1 per week |
| Supervisor | 5 Star inspection | 1 per day |
| HSE Advisor | Weekly inspection | 1 per week |
|  | 5 Star Inspection | 1 per day |

Refer:	SCEE-BS-HS-PRO-0026 Star Inspection Procedure

SCEE-BS-HS-LIS-0005 Weekly Workplace Inspection Checklist

RTIO-HSE-0077628 Health, Safety, Environment and Quality Inspection Work Practice

## Audits

To ensure compliance with SCEE and Client requirements a formal HSE audit program shall be established to validate whether the HSE systems and activities employed by SCEE and its subcontractors:

conform to agreed/ specified requirements

are suitable and effective

meet statutory/regulatory obligations

require amendment or improvement

The HSE audit program shall consist of:

Project benchmark audits (as per HSE & Quality Audit Schedule)

Site self- assessment audits (scheduled on site)

Client audits (as determined by Client Audit Schedule)

HSE system compliance audits (as part of certification audits)

Refer:	SCEE-BS-HS-PRO-0026 Star Inspection Procedure

## HSE Non-Conformance and Corrective Actions

All non-conformances or actions arising from monitoring, auditing and review activities are to be managed, tracked and closed out in accordance with the SCEE quality procedures.

All actions arising from HSE monitoring, auditing or review activities are utilised to effect change and improvement in HSE system and methods.  The Project Manager must ensure all HSE non-conformances and actions are addressed and closed out within agreed timeframes. Timeframes for the closure of corrective actions are outlined SCEE-BS-QU-PRO-0007 Non-Conformance (including Corrective and Preventative Actions) based on the classification of the actions as non-conformance, opportunity for Improvement or observation.

| Action | Description | Action Completion time frame |
| --- | --- | --- |
| Major NCR | Is a significant deviation from a specified requirement, standard, procedure, or regulation that poses a serious risk to safety, quality, the environment, or compliance. It indicates a breakdown in the management system or repeated failure to meet requirements. A Major NCR may result in project delays, legal non-compliance, or compromised integrity of the final product or process. | 7 Days |
| Minor NCR | Is a less severe deviation from a requirement or standard that does not pose an immediate risk to safety, quality, or compliance and does not significantly impact the outcome of a product or process. It usually represents an isolated incident or a procedural oversight that can be corrected without major disruption. | 14 Days |
| All other actions/tasks | Preventative/Corrective actions, tasks, observations and opportunities for improvement are proactive measures aimed at enhancing systems, processes, or performance before issues arise. These focus on identifying and eliminating potential causes of non-conformities to prevent their occurrence | 21 Days |

All identified non-conformances or corrective actions will be entered into the Project Corrective Action Register (CAR). A proposed closeout date, risk rating and a responsible person nominated for actioning the item will be identified in the CAR. This register shall be regularly reviewed by Project Management and made available to the client as required.

Refer:	SCEE-BS-QU-PRO-0007 Non-Conformance (Including Corrective and Preventative Actions)

RTIO Element 14 - Non-Conformance, Incident and Action Management

## Project Close Out Report

Project Management will compile a project HSE Close Out Report at the completion of the project. The Project HSE Close Out report reviews SCEE’s Health, Safety and Environmental performance during the project.

The report includes statistics and graphs showing lead and lag performance indicators and documents what worked well and any identified issues during the delivery of the project.

The report also summarises any identified improvement initiatives and recommendations for the continual improvement of the SCEE HSE system.

Refer: 	SCEE-BS-HS-TEM-0029 Project HSE Close Out Report Template

# Management Review

## HSE Management System Review

SCEE is committed to a program of self-auditing that endeavours to identify any non-compliance or opportunities for improvement within the HSE Management System (HSEMS).

The overall effectiveness of the HSE Management System is regularly assessed to ensure that its suitability, adequacy and effectiveness continue to meet our ongoing HSE requirements.

This project HSEMP will be reviewed within 2 months of project mobilisation and site works commencing.

The project HSEMP will be subject to periodic review and when required, amended to reflect changes in contractual or management requirements or to correct any disparity identified during monitoring, auditing or review activities.

There is an annual and event-based management system review established for the SCEE HSE Management System. These reviews are conducted by the Executive Safety Committee to determine the continuing adequacy, suitability and effectiveness of the HSEMS. The review process includes the review of audit results, incident reports, performance reports, legislative changes and relevant input from relevant stakeholders.

SCEE will also participate in Client systems audits as they apply to the SCEE scope of work.

Project Management shall support and supply the necessary resources to ensure SCEE Safety Management Systems are complied with at all levels.

Refer:	SCEE-BS-QU-PRO-0004 Management System Review

## Executive Management Review

SCEE Executive Management regularly monitor and review:

HSE performance

HSE compliance

HSE Event Reports and Investigations

The suitability and effectiveness of the HSE Management system

Whereby;

HSE events are reported to executive management as part of the notification process and investigation reports are reviewed

Statistics and data from monitoring and audit activities are reported to executive management on a monthly basis

SCEE Executive Management facilitate review of the HSE management system

Refer: 	SCEE-BS-QU-PRO-0004 Management Review

# Appendix A - Responsibility Statements

Project Manager

Reports to the Chief Operating Officer, in SCEE Head Office and is accountable for:

The overall safety performance of all SCEE workers and subcontractors under their control

Ensuring that the necessary procedures are in place to carry out all aspects of the Project Safety Management Plan

Ensuring that all requirements of this HSEMP, and HSE procedures are fully implemented and complied with

Providing adequate resources for the implementation of the HSEMP

Reporting all project progress in relation to safety to SCEE Head Office

Motivating SCEE staff to pursue their individual responsibilities within the management plans

Leading by example and conducting regular site inspections to assess SCEE’s compliance to the HSEMP requirements

Supporting SCEE Management and the Client in the resolution of all health and safety issues

Reviewing event and incident reports and minutes of safety meetings

Reviewing environmental protection activities including waste disposal

Reviewing HSE performance at regular intervals and following up any identified issues

Comply with the requirements of the RTIO Critical Risk Management process

Reviewing the implementation of the HSEMP

Being visible on site to encourage, monitor and assess all aspects of safe working procedures

Managing all sub-contractors engaged in their scope of works as SCEE workers

Ensuring the provision of adequate health and safety resources, services and facilities to effectively implement and manage the health and safety program

Reviewing Project HSE performance weekly (lag and lead indicators) with the HSE Team

Participating in Project HSE systems audits as scheduled

Attending and participating in weekly toolbox meetings

Undertaking Project field inspections

Encouraging proactive near-miss and hazard reporting by all site personnel

Addressing any hazards and risks identified during daily work activities

Immediately stopping any “Unsafe Behaviour” identified during daily work activities

I hereby certify that I have read and understood my Roles and Responsibilities as outlined in the Safety Management Plan and will comply with all requirements of the plan for the Cape Lambert A Switchgear Replacement Project.

23/04/26

Signed							Date

Site Superintendent / Supervisor

Supervisors report to the Project Manager / Project Supervisor and are responsible for:

Assisting the Project Manager and the HSE Advisor in achieving the project HSE requirements

Understanding the requirements of this HSEMP

Planning to do all work safely via compliance to the applicable procedures.

Promoting safety awareness at every opportunity

Identifying safety training needs and informing management of these needs

Ensuring personnel are suitably skilled to undertake their assigned work tasks

Promoting and conducting hazard identification and reporting

Assisting in the preparation of JHA’s/SWMSs with workers

Conducting safety inspections and initiating rectification of any issues

Participating in event and incident investigations

Facilitating prestart and toolbox meetings (which may be prepared by the HSE Advisor)

Mentoring new starters or workers who have returned to site from leave

All training undertaken on site shall be sent to the Training Department  training@scee.com.au to be uploaded in to MyPass.

Monitoring subcontractor health and safety performance

Ensuring that adequate lighting is provided and maintained at all workplaces

Ensuring there is clear demarcation of all access ways, walkways, storage areas and roads

Ensuring machine guarding is in place, adequate, and maintained

Comply with the requirements of the RTIO Critical Risk Management process

Ensuring that all hard barricading is adequate, maintained and installed around penetrations as per site requirements

Ensuring that all workers who work at height comply with relevant Work Instructions and procedures

Participating in event and incident investigations

Undertaking weekly formal inspections of work areas Undertaking daily inspections of workplaces under their control

Reviewing all JHA’s /SWMSs in the field daily

Ensuring all relevant permits are in place prior to commencement of work

Ensuring a high standard of housekeeping is maintained at all times

Removing or isolating any hazard identified during daily work activities

Immediately stopping any “Unsafe Behaviour” identified during daily work activities

Assisting and supporting the HSE Advisor with the resolution of safety issues when they arise

I hereby certify that I have read and understood my Roles and Responsibilities as outlined in the Safety Management Plan and will comply with all requirements of the plan for the Cape Lambert A Switchgear Replacement Project.

Signed							Date

Foreman/ Leading Hand

Reports to the Superintendent or Supervisor and is responsible for:

- Assisting the Superintendent / Supervisor in achieving the project Safety, Environmental and Quality requirements

- Understanding the Management Plans and the essential principals of the project practices and procedures

- Applying their professional skills in a safe manner for the proper execution of the allocated task

- Exhibiting safety awareness for their personal safety and that of their colleagues.

- Maintain very high standards of housekeeping

- Comply with the requirements of the RTIO Critical Risk Management process

- Assisting in the resolution of safety matters in a responsive and pro-active manner

- Demonstrating care for the environment including flora, fauna and culturally significant sites

- Using and caring in a proper manner for power tools, mechanical aids, plant, equipment and personal protective equipment provided

- Receiving instruction, training and other assistance and help to carry out the work to both Client and SCEE standards, adhere to policies, procedures and legislation

- Immediately stop any “Unsafe Behaviour” identified during daily work activities

I hereby certify that I have read and understood my Roles and Responsibilities as outlined in the Safety Management Plan and will comply with all requirements of the plan for the Cape Lambert A Switchgear Replacement Project.

Signed							Date

HSE Advisor

The Site HSE Advisor reports to the Project Manager/Project Supervisor and has functional reporting requirements to the HSEQ Manager.  Responsibilities and accountabilities include:

Advising Project Management in HSE matters and ensuring the project complies with all health and safety requirements

Assisting with the review and approval of the HSEMP

Developing and delivering training programs to project personnel in risk management for potential hazards on the project site

Initiating and developing safety training to assist with cultural change

Making recommendations and contributing to improving SCEE’s HSE system

Preparing HSE performance reports for SCEE management and Client Conducting and assisting with event and incident investigations

Disseminating safety bulletins on workplace events and incidents

Self-auditing of the Project Safety Management System

Conducting worker and supervisor inductions

Assisting supervisors with conducting risk assessments

Conducting daily site inspections and weekly documented site inspections

Managing hazardous materials on site

All training undertaken on site shall be sent to the Training Department  training@scee.com.au to be uploaded in to MyPass.

Comply with the requirements of the RTIO Critical Risk Management process

Management of workers compensation and rehabilitation on site, in conjunction with head office support

Attending project safety, and Committee meetings, with the findings being reported to the necessary personnel

Preparing and presenting safety presentations at weekly toolbox meetings

Monitoring and reporting on safety initiative programs

Reviewing the weekly HSE performance (lag and lead indicators) with the Project Manager

Undertaking daily informal field inspections and documenting any hazards identified

Conducting, preparing and assisting with risk assessments as required

Managing SCEE’s project HSE requirements

Assisting in the review and updating of Work Instructions, SWMS and JHA’s as required

Reviewing all permit systems prior to authorisation from the Client

Inspecting First Aid facilities and replenish stock as required

Immediately stopping any “Unsafe Behaviour” identified during daily work activities

Conducting formal JHA reviews

Implementing SCEE and Client Safety Management Systems

I hereby certify that I have read and understood my Roles and Responsibilities as outlined in the Safety Management Plan and will comply with all requirements of the plan for the Cape Lambert A Switchgear Replacement Project.

Signed							Date

Workers (including sub-contractors)

Reports to the Superintendent, Supervisor or Leading Hand and is responsible for:

- Being responsible for their own safety and health and that of others in the workplace

- Showing an understanding and demonstrating a commitment to preventing injuries, minimising damage and avoiding interruption to the organisation

- Keeping the workplace in a clean and tidy condition and not interfering with or misusing protective equipment (personal or mechanical) that has been provided or installed for the purpose of safety and health

- Using all personal and mechanical protective equipment issued or supplied as per the workplace requirements

Renewed workers licences/tickets shall uploaded into MyPass.

Comply with the requirements of the RTIO Critical Risk Management process

- Attending prestart meetings and participating in pre-start tasks

- Attending safety presentations and toolbox meetings

- Complying with the requirements for the protection of the environment

- Being responsible for their own environmental impact and that of others in the workplace

- Demonstrate environmental awareness

- Working in accordance with SCEE and requirements that have been produced to ensure the protection of the environment

- Immediately reporting all events, incidents or unsafe conditions that occur in the workplace

- Working in accordance with SCEE procedures and requirements that have been produced to ensure the safety and health of all workers and others

- Complying with the requirements of all statutory HSE legislation where applicable

- Participating in Fitness for Work testing as requested

- Adhering to permit requirements

- Assisting management in achieving the project HSE goals and objectives

- Immediately stopping any “Unsafe Behaviour” identified during daily work activities

Understanding the Heath, Safety and Environment Management Plan and Quality Plan as it relates to their position

I hereby certify that I have read and understood my Roles and Responsibilities as outlined in the Safety Management Plan and will comply with all requirements of the plan for the Cape Lambert A Switchgear Replacement Project.

Signed							Date